| Area/Activity Assessed | Operating Forklift Truck | Date | 2nd April 2026 |
| --- | --- | --- | --- |
| Assessment Completed By | T. Smith-Allen, J. Addison (MB Health & Safety Ltd) | Person(s) Consulted | D. Bryant |

| Persons Exposed | Employees |  | Contractor |  | Young Person |  | Expectant Mother |  | Visitors and/or Public |  | Trespassers |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency of Exposure | Continually |  | Hourly |  | Daily |  | Weekly |  | Monthly |  | Yearly |  |
| Duration of Exposure | Less than 1hr |  | 1-2 hrs |  | 3-4 hrs |  | 5-6 hrs |  | 7-8 hrs |  | More than 8 hrs |  |

| Probability (Prob) | 5=Very Likely, 4= Likely, 3= Quite Possible, 2= Possible, 1= Unlikely |  | 0-8 - Low risk No Action Required. 9-15 - Medium risk Ensure adequate controls are in use. 16-25 - High Risk Stop operation and implement adequate control measures |
| --- | --- | --- | --- |
| Severity (Sev) | 5=Catastrophic, 4=Major, 3=Moderate, 2=Minor, 1=Insignificant |  |  |

This Risk Assessment & Control Measures has been established following consultation with Management, Supervisory and Operational staff

Staff acknowledgment as to their understanding of the content and the control measures to be applied is included as the last page to this document

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 1. | Unauthorised use and operation of vehicle Unauthorised use of FLT could result in severe personal injury to driver or 3rd party | 5 | 4 | 20 | - Monitoring of work activities completed by Company management team who are located in the working areas throughout the day and ensure that good practice is followed. - Only trained competent staff are permitted to operate the FLT. - Specific safety, training and guidance documentation is readily available from the Company. - The vehicle is kept in a secure location and switched off when parked. - Unauthorised use of any company vehicle is deemed to be a direct violation of employee responsibility (section 7 HASAWA1974). Disciplinary action is used by the Company for any unauthorised use of vehicles. | 5 | 1 | 5 |  |

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 2. | Fall from height – accessing cab Driver could suffer personal injury or worse if they fall whilst climbing into or out of the cab | 4 | 3 | 12 | - Drivers trained to climb down from the cab, not to jump down. - Rigid handhold(s) and step(s) provided. - Steps have a slip-resistant surface. - Drivers shall visually inspect access step(s) and handhold(s) prior to use. - Vehicle flooring maintained to good standard (slip-resistant) - Drivers wear protective footwear with good grip (slip-resistant) | 4 | 2 | 8 |  |
| 3. | People movement throughout internal and external areas Staff and/or 3rd parties could suffer personal injury or worse if they were to collide with FLT moving throughout internal/external areas | 4 | 4 | 16 | - All employees and visitors are required to remain within the designated pedestrian walkways that are defined throughout the factory/warehouse areas by floor paint, whilst the FLT is in operation. - Adequate lighting levels in place where there is regular movement of vehicles, pedestrian crossings and building access points. - FLT is fitted with audible warning devices e.g. automatic reversing bleeper, flashing warning beacon. These are used at agreed locations e.g. turning into blind corner, reversing etc. - High-visibility clothing is worn by pedestrians where the risks to their safety cannot be adequately controlled by other methods. - Clear, easy read warning signs are displayed in prominent positions to inform people that FLT’s are in operation in and around the premises and external areas. - FLT drivers shall ensure that pedestrians (i.e. lorry drivers) are located away from FLT movements during loading/unloading process) - Drivers trained not to travel with a bulky load which blocks their view. | 4 | 2 | 8 |  |

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 4. | FLT tipping over due to unbalanced load Driver could suffer personal injury or worse if the FLT were to tip over | 4 | 3 | 12 | - FLT drivers are suitably trained/competent in ensuring that the load they are carrying is balanced. This training includes: - Competent use of controls, - Movement of the truck with forks or attachments in the correct travel position, laden and unladen, - Correct insertion and withdrawal of forks or other handling attachments without damage to the pallet or load, - Manoeuvring a laden and unladen truck forward and in reverse in a narrowly confined area, - Performing both a left and a right 90º turn with a loaded truck. - Fork tines are lowered when the truck is on route to required destination therefore reducing centre of gravity and tipping point of the load. - FLT is fitted with roll cage protection. - Driver knows to wear seat belt where ground conditions present a significant risk, whilst operating FLT. - All loads moved by FLT are palletised to ensure secure loading onto the tines. Pallets are inspected for suitability before materials are loaded onto them. Defective pallets are removed from use. | 4 | 2 | 8 |  |
| 5. | FLT tipping over due to excess speed Driver could suffer personal injury or worse if the FLT were to tip over | 4 | 4 | 16 | - Drivers are suitably trained/competent to operate the FLT they have been instructed to use. This training includes: - Competent use of controls, - Movement of the truck with forks or attachments in the correct travel position, laden and unladen. - Management monitor speeds and give verbal warnings if necessary. Repeat offenders are removed from using FLT and subject to company disciplinary procedures. - FLT is fitted with roll cage protection. - Driver knows to wear seat belt where ground conditions present a significant risk, whilst operating FLT. | 4 | 2 | 8 |  |

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 6. | FLT tipping over due to ground conditions of operating areas Driver could suffer personal injury or worse if the FLT were to tip over | 4 | 3 | 12 | - Flooring/ground conditions subject to ongoing review (visual inspection) by FLT drivers and staff. Any changes in conditions e.g. cracking, potholes etc. are reported immediately to the Company management team who arrange for suitable repair to be completed. - Accumulations of loose material e.g. gravel etc. on the ground are cleared on a regular basis as they are particularly hazardous to small-wheeled lift trucks. - The Company have eliminated gradients along FLT routes where possible. Drivers never allow lift trucks to be driven up or down gradients that exceed the maximum specified by the manufacturer or authorised supplier. - Drivers are suitably trained/competent to operate the FLT they have been instructed to use including never travelling or turning a FLT across a gradient. | 4 | 2 | 8 |  |
| 7. | Collision with other vehicles e.g. delivery/collection vehicles, staff cars, other estate vehicles Staff and/or other drivers could suffer personal injury or worse if their vehicle collided with a FLT moving through the external roadways | 4 | 3 | 12 | - Clear, easy read warning signs are displayed in prominent positions to inform people that FLT’s are in operation in and around the premises and external areas. - FLT is fitted with audible warning device e.g. automatic reversing bleeper, flashing warning beacon. - High-visibility clothing is worn by pedestrians where the risks to their safety cannot be adequately controlled by other methods. - FLT drivers wear high-visibility clothing at all times so they can be clearly seen when leaving the operating position of the truck. | 4 | 2 | 8 |  |
| 8. | Collision with buildings/access openings Driver could suffer personal injury or worse if the FLT collided with the building(s) or access openings | 4 | 3 | 12 | - The Company will ensure that roads and aisles have sufficient width and overhead clearance for the FLT to operate safely, whether loaded/unloaded, and if necessary, to allow other vehicles/loads to pass each other safely. - Concrete bollards positioned either side of shutter door. | 4 | 2 | 8 |  |

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 9. | FLT mechanical failure Driver could suffer personal injury if the mechanical operations of the FLT were to fail whilst in use | 3 | 3 | 9 | - Pre-use (daily) checks are completed by an appointed FLT driver before use. Any faults are reported to the Warehouse/Office Manager who will arrange for suitable repair. - Maintenance regime/contract in place and managed by the Company management team. This ensures that the FLT’s are subject to servicing and preventative maintenance on a regular basis. - Thorough Examination completed by a competent person in line with statutory requirements (LOLER) - FLT’s maintained by supplier (under contract) on a regular basis. | 3 | 2 | 6 |  |
| 10. | Parking and storage of fork truck Staff and/or 3rd parties may suffer injury due to tripping over the fork tines while it is parked up | 3 | 3 | 9 | - The Company have established suitable storage/parking areas for the FLT. These locations are away from areas where potential conflict or unauthorised access could occur. - FLT drivers are trained on safe parking procedures (e.g. park truck away from pedestrian access and gangways, fork tines placed flat on the floor with a slight downward angle so ends of fork tines are flush with the floor) | 3 | 2 | 6 |  |
| 11. | Manual Handling Driver could suffer back injury when moving the tines along mast track, removing forks from mast track etc | 3 | 3 | 9 | - Removal of fork tines is only carried out by the Company appointed service/maintenance contractor. - Manual handling training/briefing is provided for FLT drivers to remind them of best practice when moving the fork tines. - Tracks lubricated as part of the ongoing routine maintenance to assist with fork tine movement. - Location lugs and catches are maintained and serviced which makes the operation easier for the FLT driver. - Safety boots and gloves issued to all FLT drivers for use when moving the fork tines. | 3 | 2 | 6 |  |

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 12. | Checking battery fluid level Driver could suffer burns to skin and eyes due to splashes of battery acid when topping up battery | 3 | 3 | 9 | - For unsealed cells only - Driver ensures that the battery cell caps are unscrewed gently. - The hydrometer is used correctly when checking cell specific gravity. - Driver ensures that the cell caps are replaced properly after topping up. - Wearing of relevant PPE including hand, eye, face and body protection. - Eye wash available in First Aid kit. | 3 | 2 | 6 |  |
| 13. | Electrocution – connecting / disconnecting charging leads and/or battery from the FLT Driver could suffer burns to skin, eyes etc from short circuit / arcing. Danger of electric shock | 4 | 3 | 12 | - Only completed by trained/competent personnel only who follow the manufacturer’s instructions. - The driver confirms by visual inspection that the charger has been disconnected or isolated before connecting lead or leads. - The driver ensures that the FLT is turned / locked off and can’t be accidentally switched back on (keys removed) - The charging leads are connected correctly and in order – the earth leads last. | 4 | 1 | 4 |  |

| ACTION ARISING FROM RISK ASSESSMENT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Risk Rating | Action Required: | Person (s) Responsible | Target Date | Date Completed |
| 9 | Low | The FLT must be subject to an annual Thorough Examination by a suitably competent person, as defined within the Lifting Operations Lifting Equipment Regulations 1998. Copies of Thorough Examination must be held on record and available for review. | Geveko Markings | Annually |  |
| All | Low | FLT drivers to receive refresher training suitable for their knowledge and experience | Geveko Markings | 6 months from date of assessment |  |
| All | Med | This risk assessment must be communicated to all staff involved in the use of the FLT | Geveko Markings | 3 months from date of assessment |  |

Acknowledgement

The following members of staff have read this risk assessment. 
Their signatures are confirmation that they have read and understood all which is within its contents.

| Name | Signature | Date | Name | Signature | Date |
| --- | --- | --- | --- | --- | --- |