| Area/Activity Assessed | General Warehouse Activities | Date | 2nd April 2026 |
| --- | --- | --- | --- |
| Assessment Completed By | T. Smith-Allen, J. Addison (MB Health & Safety Ltd) | Person(s) Consulted | D. Bryant |

| Persons Exposed | Employees |  | Contractors |  | Young Person |  | Expectant Mother |  | Visitors and/or Public |  | Trespassers |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency of Exposure | Continually |  | Hourly |  | Daily |  | Weekly |  | Monthly |  | Yearly |  |
| Duration of Exposure | Less than 1hr |  | 1-2 hrs |  | 3-4 hrs |  | 5-6 hrs |  | 7-8 hrs |  | More than 8 hrs |  |

| Probability (Prob) | 5= Very Likely, 4= Likely, 3= Quite Possible, 2= Possible, 1= Unlikely | Low | 0-8 | Low risk no action required |
| --- | --- | --- | --- | --- |
|  |  | Medium | 9-15 | Medium risk ensure adequate controls are in use |
| Severity (Sev) | 5= Catastrophic, 4= Major, 3= Moderate, 2= Minor, 1= Insignificant | High | 16-25 | High risk stop operation and implement adequate control measures |

This Risk Assessment and identified Control Measures has been established following consultation with Management, Supervisory and Operational staff

Staff acknowledgment as to their understanding of the control measures to be applied is included as the last page to this document

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 1. | Manual handling of materials Staff risk injuries or back pain from handling heavy/bulky objects, e.g. deliveries of product, picking and packing | 3 | 4 | 12 | - Manual handling of materials is avoided wherever possible. - The distance that materials have to be moved to/from is minimised to avoid excessive carrying/lifting etc. - The use of mechanical aid is the preferred choice of moving items however where it is unavoidable the Company ensure that relevant staff are trained in good manual handling techniques. - Staff advised never to exceed their own personal capability when lifting/moving items. - FLT/pallet truck used to transport boxes and other heavy items when collecting deliveries etc. - Heavy items are stored and are accessible at the appropriate height. - Lightest items are stored on high shelves. | 3 | 3 | 9 |  |
| 2. | Chemical Staff could suffer ill health as a result of the handling of hazardous substances | 4 | 3 | 12 | - Material Safety Data Sheets (MSDS) available for those substances identified as being hazardous. - Access to the chemical store is restricted to warehouse personnel. - Suitable and sufficient warning signs displayed. - Products stored are for re-sale only and as such presents no significant risk other than general manual handling risk. - Emergency spill clear-up kits provided. | 4 | 2 | 8 |  |

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 3. | Use of Forklift Truck (FLT) Staff and 3rd parties could suffer severe personal injury or worse as a result of FLT use | 4 | 4 | 16 | - Drivers are suitably trained/competent to operate the FLT they have been instructed to use. - The Company ensures a specific risk assessment is completed (re. 2.2.2 – ‘Operating Forklift Truck’) - The assessment considers the provision of preventative measures, maintenance and inspection, and provision of staff training. - Daily checks are completed by an appointed FLT driver before use. Any faults are reported to the Warehouse/Office Manager who will arrange for suitable repair. - Maintenance regime/contract in place and managed by the Company management team. This ensures that the FLT’s are subject to servicing and preventative maintenance on a regular basis. - Thorough Examination completed by a competent person in line with statutory requirements (LOLER) - FLT’s maintained by supplier (under contract) on a regular basis. | 4 | 2 | 8 |  |
| 4. | Working at height – picking from racking, storing stock, maintenance activities etc. Staff could suffer severe personal injury as a result of a fall from height | 4 | 3 | 12 | - Appropriate equipment, e.g. suitable step ladder/warehouse steps, are provided and staff know how to use them safely. - All access equipment used by staff will meet the relevant British Standard. In addition, an inspection of condition will be undertaken before each use. Any considered to be unfit for purpose will be withdrawn from use, i.e. damaged rungs, guard rails etc. - Step ladders/warehouse steps will be sited on a firm and level surface ensuring there are no overhead obstructions. - The user will check that the steps have been erected correctly and are stable. - The user will not ‘over-reach’ to gain access. - Step ladders/warehouse steps will be dismounted prior to re-siting. - Step ladders will be used for access only and will not be used as a ‘workstation’. - Warehouse steps SSOW. - Formal monthly inspection undertaken of the warehouse steps and the results documented. | 4 | 2 | 8 |  |

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 5. | Health of workers – work related stress Staff could suffer ill health as a result of pressure of work demands, lack of job control (e.g. no control over orders to be picked/packed), insufficient support from colleagues, not knowing their role, poor relationships, badly managed change, bullying or verbal abuse from customers | 3 | 3 | 9 | - Work plans & objectives discussed and agreed. - Staff are trained how to do their job effectively. - Flexible working patterns. - Management control. - Regular team meetings. - Staff training available to ensure skills updates as necessary. - Staff encouraged to use their initiative and develop new skills. - Managers to take seriously any issues that are brought to them. and try to resolve quickly. - Company support mechanisms in place. - Team structure and responsibilities are clearly defined. - Arranged socials. - Honest and open communication with all staff. - Staff can speak confidentially to a supervisor/manager if they are feeling unwell or ill at ease about things at work. | 3 | 1 | 3 |  |
| 6. | Use of Stanley knife Staff could suffer injury to themselves or others as a result of improper use or poorly maintained equipment | 3 | 4 | 12 | - Knives have retractable blades. - Knives selected for the task(s) have a handle which allows a firm and comfortable grip - Spare knives and blades are available. - Suitable storage arrangements are in place to prevent injuries to both the knife user and others, e.g. situations where knives are left lying loose on work benches/surfaces. - Used blades are placed in a suitable receptacle/or adequately protected for final disposal. - Users ensure they have enough free space to move freely and allow them to operate in a safe manner without endangering themselves or others. - Adequate lighting levels are provided. - Users are competent in the general use, care and maintenance of hand knives (including typical accidents, cutting away from the body and the danger of blunt knives) | 3 | 2 | 6 |  |

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 7. | Use of Pallet Truck Staff and 3rd parties could suffer personal injury as a result of improper use | 3 | 3 | 9 | - The condition of the equipment is checked before use, if it shows signs of damage or excessive wear it is reported to the Warehouse/Office Manager. - The equipment is used on level areas able to take the combined weight of the load and the equipment. - The equipment is not used on slopes, soft ground or where there is a risk of subsidence. - The equipment will not be used wear there are slipping agents, i.e. ice or oil deposits. - The operator wears protective clothing, i.e. gloves and safety footwear. - Personnel in the immediate work area shall be warned of the operation taking place. - The equipment will not be used to carry unsecured loads. - The Safe Working Load (SWL) is not exceeded. - The operator will ensure that the forks are centered under the load. - The operative shall ensure that the load is balanced, stable and personnel stand clear of the raised load. - If the load is such a height that the operator cannot see over it, the pallet truck will be pulled to its destination. - When loading and unloading the operative shall ensure the weight is evenly distributed on the pallet so as not to over balance the load. - The operative will not allow the truck to travel at speed or change its direction abruptly. - When moving loads the operator keeps to a sensible pace. - The operator shall work in such a way they keep their hands and feet from under the forks and any load. Hands are placed on the control handles when manoeuvring the equipment. - The operator positions them self so that they can control the pallet truck while staying clear of the raised load. - The equipment shall not be used to carry personnel. | 3 | 2 | 6 |  |

| ACTION ARISING FROM RISK ASSESSMENT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Risk Rating | Action Required: | Person (s) Responsible | Target Date | Date Completed |
| 1 | Med | The company must ensure that relevant staff are trained in good manual handling techniques. This could include the use of a suitable Manual Handling Awareness e-learning module to support the in-house training. | Geveko Markings Management | Annually |  |
| 2 | Low | A formal review and CoSHH assessment(s) to be completed for the products stored, to ensure that current controls are suitable for the exposure faced by employees and any 3rd party who may come into contact with them | Geveko Markings Management supported by MBHS | 6 months from date of assessment |  |
| All | Med | This risk assessment must be communicated to all staff involved in this activity | Preformed Markings supported by MBHS | 3 months from date of assessment |  |

Acknowledgement

The following members of staff have read this risk assessment.

Their signatures are confirmation that they have read and understood all which is within its contents.

| Name | Signature | Date | Name | Signature | Date |
| --- | --- | --- | --- | --- | --- |