| Area/Activity Assessed | General installation (demonstration & training) | Date | 2nd April 2026 |
| --- | --- | --- | --- |
| Assessment Completed By | T. Smith-Allen, J. Addison (MB Health & Safety Ltd) | Person(s) Consulted | D. Bryant |

| Persons Exposed | Employees |  | Contractors |  | Young Person |  | Expectant Mother |  | Visitors and/or Public |  | Trespassers |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency of Exposure | Continually |  | Hourly |  | Daily |  | Weekly |  | Monthly |  | Yearly |  |
| Duration of Exposure | Less than 1hr |  | 1-2 hrs |  | 3-4 hrs |  | 5-6 hrs |  | 7-8 hrs |  | More than 8 hrs |  |

| Probability (Prob) | 5= Very Likely, 4= Likely, 3= Quite Possible, 2= Possible, 1= Unlikely | Low | 0-8 | Low risk no action required |
| --- | --- | --- | --- | --- |
|  |  | Medium | 9-15 | Medium risk ensure adequate controls are in use |
| Severity (Sev) | 5= Catastrophic, 4= Major, 3= Moderate, 2= Minor, 1= Insignificant | High | 16-25 | High risk stop operation and implement adequate control measures |

This Risk Assessment and identified Control Measures has been established following consultation with Management, Supervisory and Operational staff

Staff acknowledgment as to their understanding of the control measures to be applied is included as the last page to this document

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 1. | Manual handling of materials Staff risk injuries or back pain from handling heavy/bulky objects, e.g. carrying product/equipment to installation location | 3 | 4 | 12 | - Manual handling of materials/equipment is avoided wherever possible. - Materials/equipment will be transported by vehicle to a position as close as possible to the point of final use. - The distance that materials/equipment have to be moved to/from is minimised to avoid excessive carrying/lifting etc. - The use of mechanical aid is the preferred choice of moving items however where it is unavoidable the Company ensure that relevant staff are trained in good manual handling techniques. - Staff advised never to exceed their own personal capability when lifting/moving items. - Where possible the load will be broken down into smaller loads for easier transportation. - Appropriate PPE is supplied and worn. | 3 | 2 | 6 |  |

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 2. | Use of Stanley knife Staff could suffer injury to themselves or others as a result of improper use or poorly maintained equipment | 3 | 3 | 9 | - Knives have retractable blades. - Knives selected for the task(s) have a handle which allows a firm and comfortable grip - Spare knives and blades are available. - Suitable storage arrangements are in place to prevent injuries to both the knife user and others, e.g. situations where knives are left lying loose on work surfaces, floor. - Used blades are placed in a suitable receptacle/or adequately protected for final disposal. - Users ensure they have enough free space to move freely and allow them to operate in a safe manner without endangering themselves or others. - User shall ensure that the knife is only used where adequate lighting levels are provided. - Users are competent in the general use, care and maintenance of hand knives (including typical accidents, cutting away from the body and the danger of blunt knives) | 3 | 2 | 6 |  |
| 3. | Contact with substances/materials Staff could suffer occupational dermatitis, Asthma and other ill health effects caused by exposure to substances | 3 | 3 | 9 | - Level of exposure to substances (primer etc.) is minimal. - Material Safety Data Sheets (MSDS) and corresponding CoSHH risk assessments completed for those substances identified as being hazardous. - Appropriate PPE readily available for staff when handling materials and substances. - The Company ensure that all Staff that may use or come into contact with materials and substances are made aware of control measures to be applied. - No history of ill health effects suffered by any member of staff. | 3 | 2 | 6 |  |
| 4. | Burns Staff and 3rd parties could suffer severe burns as a result of contact with burner flame or hot surfaces | 4 | 3 | 12 | - A purpose designed burner lance ensures that hand(s) are kept at a suitable distance from the flame. - Non-essential personnel are excluded from the immediate working area when the burner is in use. - Staff allow the lance to cool down before transportation/storage. | 4 | 2 | 8 |  |

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 5. | Exposure to fumes/vapours during installation process Exposure to harmful vapours can result in ill health | 3 | 3 | 9 | - Work process restricts airborne contaminants to localised area and not a significant issue for non-operational personnel. - Staff shall ensure that there is adequate general ventilation to prevent the build-up of harmful vapours. | 3 | 2 | 6 |  |
| 6. | Accidental ignition due to poor housekeeping, work area design, or inspection Staff and 3rd parties could suffer severe personal injury or worse as a result of not following safe working practises | 5 | 3 | 15 | - The immediate work area is cleared of any flammable materials and or suitable fire screens or blankets put in place to prevent contact between flame and flammables. - Gas hoses, valves are checked before and after use. - A work site inspection is carried out after completion of hot work to ensure there are no residual hot spots or smouldering fires that could escalate to a large fire. | 5 | 1 | 5 |  |

| ACTION ARISING FROM RISK ASSESSMENT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Risk Rating | Action Required: | Person (s) Responsible | Target Date | Date Completed |
| All | Med | This risk assessment must be communicated to all staff involved in this activity | Geveko Markings Management | 3 months from date of assessment |  |

Acknowledgement

The following members of staff have read this risk assessment.

Their signatures are confirmation that they have read and understood all which is within its contents.

| Name | Signature | Date | Name | Signature | Date |
| --- | --- | --- | --- | --- | --- |