Name:………………………………………………		Company: ………………………………………………

Signed: ………………………………………………	Date: ………………………………………………

| Commitment to Safety | The Management Team at the Company are committed to establishing a safe pace of work for everyone whilst they are on site. No-one is expected to work in conditions or areas that they believe are unsafe and ask that everyone working or involved on the project help to create an ongoing positive attitude to health and safety. |  |
| --- | --- | --- |
| Project Management | You will be allocated a point of contact whilst at our premises and their details and location will be provided to you. Company Contact: |  |
| Risk Assessment | You will be required to provide a suitable risk assessment for the works that you are undertaking. The Company can issue you with a basic risk assessment form for you to complete if you don’t have one. This document is your responsibility to complete as fully as possible, the Company will not be responsible for the information you include. |  |
| Particular Risks & Controls | Vehicle movements | Ensure that you are clearly visible to vehicle drivers who will be constantly accessing the site from the estate roadways. Please keep away from vehicle routes, especially when they are reversing. Use of Forklift Trucks is a key element of the work processes we undertake both inside and outside of the buildings. Please remain within the pedestrian walkways in each building (where possible). High Vis clothing (vest, jacket) is required when you are in the yard area – no exceptions. |
|  | Utilities / Services | Please liaise with the Management Team if you require services or utilities isolated. |
|  | Lighting | Please ensure that you have any additional task lighting that may be required for the area/activity you are completing |
|  | Work at height | No-one is allowed to work on any platform that has an open or leading edge – make sure that a handrail, intermediate rail and toe boards are in place to prevent falls. It is expected that you will select the most appropriate access equipment for the work you are undertaking and that it will be in good order, please check before you use it. |
|  | Environmental Consideration | Please show consideration to the neighbouring commercial businesses and the residential housing that is in close proximity. Adhere to the speed limits when leaving our site and the estate itself. |
| Signing In/Out | All personnel either working or visiting the site MUST sign the visitor register on arrival and when departing so an accurate record of who is on site can be maintained. |  |
| Welfare Provision | The Company welfare facilities are available for your use whilst you are on site. This includes all toilet/washing facilities and kitchen/canteen areas. Please ensure that they are kept clean and tidy at all times. Do not allow food waste to accumulate – bin it! |  |

| Emergency Arrangements | First Aid | A first aid kit and trained personnel are available on site. If you need help, please ask |
| --- | --- | --- |
|  | Accident/Incident Reporting | If an accident or incident occurs, please ensure that your appointed Company contact is notified. Accidents (that lead to injury) must be recorded following the company procedure. Incidents (that don’t involve injury) will be recorded. Information on accidents/incidents will be reviewed and used to make improvements where possible. |
|  | Fire | In the event of the fire alarm being activated you will hear a loud noise. Turn off any equipment/tools you are using. Leave your working area via the nearest exit point and congregate at the assembly point. Any person not answering to their name will be presumed missing and their details passed to the emergency services on arrival. |

And Most Importantly

| Responsibilities | Drive safely to and from the site – don’t speed, if you’re running late, make a call to your Supervisor to let him know. Health and safety is EVERYONE’s responsibility – not just the management. Never ignore any restrictions to areas that have been issued – they’re in place for a reason. Don’t ignore something or an issue that looks unsafe – report it Challenge the behaviour of others – if it looks unsafe, ask them to stop and consider what they’re doing. Complacency can be dangerous! Don’t compromise your health and safety by cutting corners or taking short cuts. Think about how the work you’re doing impacts on others – keep tools, equipment and materials in a tidy manner. If in doubt – STOP!! Seek advice from the Curran Packaging management team. |
| --- | --- |