| Area/Activity Assessed | Use of hand held tools | Date |  |
| --- | --- | --- | --- |
| Assessment Completed By |  | Person(s) Consulted |  |

| Persons Exposed | Employees |  | Contractor |  | Young Person |  | Expectant Mother |  | Visitors and/or Public |  | Trespassers |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency of Exposure | Continually |  | Hourly |  | Daily |  | Weekly |  | Monthly |  | Yearly |  |
| Duration of Exposure | Less than 1hr |  | 1-2 hrs |  | 3-4 hrs |  | 5-6 hrs |  | 7-8 hrs |  | More than 8 hrs |  |

| Probability (Prob) | 5= Very Likely, 4= Likely, 3= Quite Possible, 2= Possible, 1= Unlikely | Low | 0-8 | Low risk no action required |
| --- | --- | --- | --- | --- |
|  |  | Medium | 9-15 | Medium risk ensure adequate controls are in use |
| Severity (Sev) | 5= Catastrophic, 4= Major, 3= Moderate, 2= Minor, 1= Insignificant | High | 16-25 | High risk stop operation and implement adequate control measures |

|  | Task / Hazard | Initial |  |  | Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
|  | Ergonomic Operators could suffer Musculo-skeletal injuries as a result of using heavy equipment, repetitive use, or poor design of workstation | 3 | 3 | 9 | Operator ensures that there is adequate room to use the tool /equipment safely and without restriction on movement. The use of heavy equipment is minimised as far as is reasonable practicable (e.g. job rotation) Jigs and suspension systems are used to assist the handling of heavy equipment. Workstations are designed and work organised to avoid uncomfortable postures. Safety footwear worn unless feet protected by other means (e.g. sitting at a bench) | 3 | 2 | 6 |  |
|  | Equipment failure leading to potential impact on Operator e.g. electrocution Operator and others could suffer electrical shock, burns or personal injury if equipment is faulty | 4 | 3 | 12 | Pre-use check completed by Operator who can raise fault report if concerned on any aspect of tool/equipment maintenance. Tool/equipment is subject to routine maintenance regime by in house staff who are competent and experienced. An in line Residual Current Device (RCD) is provided and used. Electrical outlets are RCD protected. Portable electrical equipment and power tools are maintained, inspected and tested at 12 monthly intervals (PAT) | 4 | 2 | 8 |  |

|  | Unauthorised use and operation of equipment. Operator could suffer personnel injury or cause injury to others as a result of using a tool or piece of equipment that they are unfamiliar with |  | 3 | 4 | 12 |  | Monitoring of work activities completed by nominated supervisor(s) who are located in the working areas throughout the day and ensure that good practice is followed. Only trained competent staff utilise equipment and a Competency Matrix has been established by the Company to ensure clarity of who is authorised to use named pieces of equipment. Specific safety, training, equipment authorisation and guidance documentation is readily available from the Company. Power tools are securely stored when not in use. Unauthorised use of any tool or piece of equipment is deemed to be a direct violation of employee responsibility (section 7 HASAWA1974). Disciplinary action is used by the Company for any unauthorised use of tools and equipment. | 3 | 2 | 6 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Entrapment/Entanglement leading to cuts, abrasions Staff could suffer personnel injury if they were to become entangled in the equipment whilst in operation |  | 3 | 4 | 12 |  | Two-handed operation prevents accidental contact with rotating parts. Fixed guarding restricts contact with moving parts as far as it reasonable practicable. Guards shall be checked before use to ensure adequate protection against dangerous parts of tool/equipment. Loose clothing and jewellery, including rings with large protrusions, are not worn. Long hair is required to be tied back. Equipment is isolated from power source for tooling change (i.e. router/drill bit). ‘Hold-to-run’ controls. Operators are appropriately trained and competent for the use of the tool/equipment. Equipment is isolated if tool jamming or binding occurs. Competency Matrix has been established by the Company to ensure clarity of who is authorised to operate the equipment. Monitoring of work activities completed by nominated supervisor(s) who are located in the working areas throughout the day and ensure that good practice is followed. Specific safety, training, equipment authorisation and guidance documentation is readily available from the Company. | 3 | 3 | 9 |  |
|  |  | Ejection of materials Operators and others could suffer personnel injury if they are struck by materials being ejected from the equipment | 4 | 3 | 12 | Non-essential personnel are excluded from the immediate working area when the tool/equipment is in use. Guarding is appropriate to prevent materials being ejected into the operator or the surrounding area. Safety eyewear worn by operator. Persons nearby are advised of hazard. Monitoring of work activities completed by nominated supervisor(s) who are located in the working areas throughout the day and ensure that good practice is followed. Specific safety, training, equipment authorisation and guidance documentation is readily available from the Company. |  | 4 | 2 | 8 |  |
|  |  | Slips, trips and falls Staff could suffer personnel injury if a slip, trip or fall occurs | 3 | 3 | 9 | Staff are aware of the need for good housekeeping around the working area. This ensures that floors are kept free from obstructions and potential trip hazards. Floor surfaces around the work station/area are subject to ongoing visual inspections by Staff throughout the working day. Trailing leads are routed away from walkways or are covered to reduce trip hazard. Monitoring of work activities and working areas completed by nominated supervisor(s) throughout the day and ensure that good practice is followed. |  | 3 | 2 | 6 |  |
|  |  | Contact with electricity Operator could suffer severe personal injury or worse if they were to come into contact with electricity | 4 | 4 | 16 | Condition of lead and plug checked before use. Trailing leads are routed away from walkways or are covered to reduce possibility of accidental damage. 110v or battery tool. An in line Residual Current Device (RCD) is provided and used. Portable electrical equipment and power tools are maintained, inspected and tested at 12 monthly intervals (PAT) Tools not used where water is present. |  | 4 | 2 | 8 |  |

|  | Exposure to noise Operator and 3rd parties could suffer work based noise induced hearing loss if prolonged exposure to noisy equipment | 3 | 4 | 12 | Appropriate steps are taken to reduce the noise to as low as reasonably practicable by ensuring appropriate maintenance of ancillary items such as drill bits, cutters etc., and where possible consideration is given to the purchasing of equipment with built-in noise reduction features. Suitable hearing protection is made available (free of charge) to all operators and others exposed to noise levels over 80 dB(A) The use of the correct hearing protection by operators and others exposed to noise levels of over 85 dB(A) is enforced. Persons nearby are advised of hazard. Audiometry testing and monitoring completed by appointed Occupational Health specialist. | 3 | 2 | 6 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Exposure to dust and airborne particles created during use Operator and 3rd parties could suffer personal injury or ill health as a result of coming into contact with materials and substances via touching, inhalation, ingestion, absorption. Creation of dust may also reduce visibility | 3 | 4 | 12 | Work process restricts airborne contaminants to localised area and not a significant issue for non-operational personnel. Integrated dust bag. Built in extractor ports connected to external extraction system. Suitable RPE used as secondary means of removal of airborne contaminants. Dry sweeping of dust is not permitted. Respiratory function testing completed by appointed Occupational Health specialist. Close-fitting safety goggles worn. Work ceases if visibility is noticeably reduced. | 3 | 2 | 6 |  |

|  | Vibration Operator could suffer Hand Arm Vibration Syndrome (HAVS) or Carpal Tunnel Syndrome (CTS) as a result of excessive exposure to vibrating tools/equipment | 3 | 3 | 9 | Tool/equipment is properly stored, maintained and used according to manufacturer’s instructions, including any ancillary items such as drill bits, cutters etc. The time individuals use the equipment is minimised (e.g. job/task rotation) The need for high manual effort to grip, push or pull equipment is minimised Work piece well supported and stable during machining process, use of jigs, clamps, vices, vacuum tables where appropriate. Workstations are designed and work organised to avoid uncomfortable postures. Operators are not exposed to levels above the EAV (2.5m/s2 A(8)) or ELV (5m/s2 A(8)) If it is likely the Operator will be exposed above the EAV the company will introduce a programme of controls to eliminate risk, or reduce exposure to as low a level as is reasonably practicable. If it is likely the Operator will be exposed above the ELV immediate action will be taken to reduce their exposure below the limit value. Health surveillance undertaken for those persons at risk from vibration. | 3 | 1 | 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compressed air line detaching or bursting Operator or 3rd Parties could suffer eye/face injury as a result of fault with air line | 3 | 3 | 9 | Condition and connections of air-line checked before use. Operator to ensure that air-line is not on a traffic route (pedestrian or vehicular) | 3 | 2 | 6 |  |

|  | Working within a confined space Operatives and other site users - could suffer from smoke inhalation, burns and/or serious personal injury if trapped on site. | 4 | 4 | 16 | A competent person must assess the area and decide if it is a confined space as defined by the regulations. If it is, can the work be re-planned so as to avoid the need to enter. Entry into the working area is controlled by a permit system maintained by the Client site management team Site specific method statement has been established detailing the method of work, equipment required, training and competence requirements of those people working in the confined space. PPE to be worn (overalls, head protection, safety footwear, hand protection and FFP3 dust mask). Materials being used by TP operatives in a confined space are defined as solvent free by the product manufacturer and no “forced ventilation” is required whilst work is being completed. TP ensure that no lone working is conducted during the completion of the project Access/egress route is maintained ensuring that if emergency extraction of TP operatives is required no additional equipment (tripod, winch etc) is required | 4 | 2 | 8 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| ACTION ARISING FROM RISK ASSESSMENT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Risk Rating | Action Required: | Person (s) Responsible | Target Date | Date Completed |
| 4 | Medium | This risk assessment must be communicated to all staff involved in the use of the tool/equipment | Company Name supported by MBHS | 6 months from date of assessment |  |

Risk Assessment Acknowledgement

The following operatives have read this risk assessment. 
Their signatures are confirmation that they have read and understood all which is within its contents.

| Name | Signature | Date | Name | Signature | Date |
| --- | --- | --- | --- | --- | --- |