| Persons Exposed | Employees |  | Contractor |  | Young Person |  | Expectant Mother |  | Visitors and/or Public |  | Trespassers |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency of Exposure | Continually |  | Hourly |  | Daily |  | Weekly |  | Monthly |  | Yearly |  |
| Duration of Exposure | Less than 1hr |  | 1-2 hrs |  | 3-4 hrs |  | 5-6 hrs |  | 7-8 hrs |  | More than 8 hrs |  |

| Probability (Prob) | 5= Very Likely, 4= Likely, 3= Quite Possible, 2= Possible, 1= Unlikely | Low | 0-8 | Low risk no action required |
| --- | --- | --- | --- | --- |
|  |  | Medium | 9-15 | Medium risk ensure adequate controls are in use |
| Severity (Sev) | 5= Catastrophic, 4= Major, 3= Moderate, 2= Minor, 1= Insignificant | High | 16-25 | High risk stop operation and implement adequate control measures |

|  | Task / Hazard | Initial |  |  | Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
|  | Unauthorised use and operation of equipment. Operator could suffer severe personnel injury or cause injury to others as a result of using a piece of equipment that they are unfamiliar with. | 4 | 4 | 16 | Monitoring of work activities completed by nominated supervisor(s) who are located in the working areas throughout the day and ensure that good practice is followed. Only trained competent staff utilise equipment and a Competency Matrix has been established by the Company to ensure clarity of who is authorised to use named pieces of equipment. Specific safety, training, equipment authorisation and guidance documentation is readily available from the Company. The oxygen and dissolved acetylene package & empty cylinders are stored in a secure location. Unauthorised use of any tool or piece of equipment is deemed to be a direct violation of employee responsibility (section 7 HASAWA1974). Disciplinary action is used by the Company for any unauthorised use of tools and equipment. | 4 | 1 | 4 |  |

|  | Incorrect Storage Staff and 3rd parties could suffer severe personal injury or worse as a result of an external fire or heat impacting on the oxygen and dissolved acetylene package. Empty cylinder abuse | 4 | 3 | 12 | Oxygen and acetylene cylinder valves are closed. Empty cylinders are treated as though full, applying the same precautions. Cylinder key is removed. Empty cylinders are stored in designated area. Empty cylinders are returned to gas supplier or agent as soon as possible. The torch, hoses and pressure regulator are vented, as per equipment instructions. The pressure adjusting screw on regulators is backed-off. Gas hoses are rolled up then hoses and torch are stored on the appropriate stowage point on the package. The oxygen and dissolved acetylene package & empty cylinders are stored in a secure location away from flammable/combustible materials and sources of ignition The oxygen and dissolved acetylene package is stored in a secure area with access limited to trained and authorised persons. Warning notices in accordance with the Safety Signs and Signals Regulations e.g. 'Smoking and Naked Flames Forbidden' and 'No Access for Unauthorised Persons', are displayed. The number of cylinders kept indoors is reduced to as low a number as is practicable. Where possible cylinders are stored in dedicated outdoor area, and cylinders brought indoors as needed. | 4 | 1 | 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unknown quantity of gas cylinders in premises Inadequate control of gas stored on site could impact the general safety of staff and 3rd parties, in particular Fire and Rescue Service personnel due to a lack of knowledge of what gas cylinders are present in the fire zone | 3 | 3 | 9 | An up-to-date list of gas cylinders stored on site is maintained (location and quantity) A list is displayed in an easily accessible location. | 3 | 1 | 3 |  |

|  | Manual handling Operator could suffer personal Injury due to incorrect handling. Poor handling could also result in property damage |  | 3 | 3 | 9 |  | Large cylinders are moved using a suitable trolley or assistance is sought. Operators are advised to not drop gas cylinders and never try to catch a falling cylinder. Cylinders are secured in a suitable cylinder trolley, cylinder support bracket or placed in a cylinder pen. | 3 | 1 | 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Incorrect use of oxygen and dissolved acetylene package and equipment Operator and 3rd parties could suffer severe personnel injury or worse due to Fire resulting from the misuse of Oxyacetylene Pack e.g. regulator pressures set wrongly, incorrect torch light up/shutdown procedure used causing a back fire or flashback. | 5 | 4 | 20 | All users are trained in: - Properties of the gases; - The safe use of the equipment; - Precautions to be taken; - The use of fire extinguishers; - Emergency Procedures including the BOC document entitled "KEY ACTIONS for dealing with cylinders in the event of fire". Pre-use equipment checks are carried out on the oxygen and dissolved acetylene package and equipment. Procedures/work instructions for the safe use of the equipment are provided. The following work rules have been created and are followed: - FORBIDDING the use of oxygen for dusting down work benches, machinery or clothing; - FORBIDDING the hanging up of the welding or cutting torch whilst still alight. Gas hoses are positioned out of the line of fire from the flame or metal spatter. The correct pressures and nozzle size for the job are used. The acetylene pressure does not exceed 0.62bar (9psi) Causes of flashback or backfire are investigated & any damage or faults to the equipment is identified (particularly the nozzle) Appropriate PPE is provided and worn, i.e. welding gloves, leather apron, suitable coverall, foot protection, eye and head protection. |  | 5 | 2 | 10 |  |

|  | Use of faulty or incorrect equipment (regulator, flashback arrestors, torch, hoses etc.) resulting in fire Operator and 3rd parties could suffer severe personnel injury or worse resulting from the use of faulty or incorrect equipment (regulator, flashback arrestors, torch, hoses, etc.) | 5 | 4 | 20 | Only equipment supplied by a reputable equipment supplier is used, which has been designed for the intended gas service i.e. oxygen and acetylene compatible. Refer to BCGA CP7 - routine inspection and maintenance. Hoses are manufactured to BSEN 559 and correctly coloured for the gas service i.e. blue for oxygen and red for acetylene. The hose assembly is to EN ISO 8330. Flash back Arrestors are fitted to the outlet of both oxygen and acetylene pressure regulators in accordance with legislative recommendations (DSEAR) Re-settable Flash Back Arrestors are replaced at the manufacturers recommended intervals. Disposable Flash Back Arrestors are replaced after a flashback or backfire. Non Return Valves are fitted on the hose end connecting to the blowpipe or torch. Regulators are refurbished or replaced every 5 years. The following work rule has been created and is followed: - FORBIDDING the forcing of a regulator connection onto the cylinder (acetylene is left hand thread and oxygen is right hand thread) Only equipment designed for oxygen is used and such equipment is appropriately labelled. Clean hands or gloves are used when assembling oxygen equipment e.g. attaching a regulator to a new cylinder or attaching the hoses to the blowpipe or torch. Oil or Grease is not used during assembly. Defective equipment is replaced before attempting to use the oxygen and dissolved acetylene package. | 5 | 1 | 5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Accidental ignition due to poor housekeeping, work area design, or inspection Operator and 3rd parties could suffer severe personal injury or worse as a result of not following safe working practises | 5 | 3 | 15 | The immediate work area is cleared of any flammable materials and or suitable fire screens or blankets put in place to prevent contact between sparks and flammables. A work site inspection is carried out after completion of hot work to ensure there are no residual hot spots or smouldering fires that could escalate to a large fire. | 5 | 1 | 5 |  |

| ACTION ARISING FROM RISK ASSESSMENT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Risk Rating | Action Required: | Person (s) Responsible | Target Date | Date Completed |
| 5 | Med | This risk assessment must be communicated to all staff involved in the maintenance and use of the equipment | Company Name supported by MBHS | 3 months from date of assessment |  |

Risk Assessment Acknowledgement

The following operatives have read this risk assessment. 
Their signatures are confirmation that they have read and understood all which is within its contents.

| Name | Signature | Date | Name | Signature | Date |
| --- | --- | --- | --- | --- | --- |