| Area/Activity Assessed | Operating Fork Lift Truck | Date |  |
| --- | --- | --- | --- |
| Assessment Completed By |  | Person(s) Consulted |  |

| Persons Exposed | Employees |  | Contractor |  | Young Person |  | Expectant Mother |  | Visitors and/or Public |  | Trespassers |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency of Exposure | Continually |  | Hourly |  | Daily |  | Weekly |  | Monthly |  | Yearly |  |
| Duration of Exposure | Less than 1hr |  | 1-2 hrs |  | 3-4 hrs |  | 5-6 hrs |  | 7-8 hrs |  | More than 8 hrs |  |

| Probability (Prob) | 5= Very Likely, 4= Likely, 3= Quite Possible, 2= Possible, 1= Unlikely | Low | 0-8 | Low risk no action required |
| --- | --- | --- | --- | --- |
|  |  | Medium | 9-15 | Medium risk ensure adequate controls are in use |
| Severity (Sev) | 5= Catastrophic, 4= Major, 3= Moderate, 2= Minor, 1= Insignificant | High | 16-25 | High risk stop operation and implement adequate control measures |

|  | Task / Hazard | Initial |  |  | Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
|  | People movement throughout internal and external areas. Staff and/or 3rd parties could suffer personal injury or worse if they were to collide with FLT moving throughout internal/external areas | 4 | 4 | 16 | The Company have defined and clearly marked pedestrian routes and crossing places. Adequate lighting levels in place where there is regular movement of vehicles, pedestrian crossings and building access points. FLT are fitted with audible warning devices e.g. automatic reversing bleepers, flashing warning beacons. These are used at agreed locations e.g. turning into blind corner, reversing etc. Visibility aids are used where required e.g. convex mirrors at blind corners. High-visibility clothing is worn by pedestrians where the risks to their safety cannot be adequately controlled by other methods. Clear, easy read warning signs are displayed in prominent positions to inform people that FLT’s are in operation in and around the premises and external areas. FLT drivers utilise a banksman to manage pedestrians when loading vehicles (ensuring that lorry drivers are located away from FLT movements during the loading/unloading process) | 4 | 3 | 12 |  |

|  | FLT tipping over due to unbalanced load Driver could suffer personal injury or worse if the FLT were to tip over | 4 | 3 | 12 | FLT drivers are suitably trained/competent in ensuring that the load they are carrying is balanced. This training includes: - Competent use of controls, - Movement of the truck with forks or attachments in the correct travel position, laden and unladen, - Correct insertion and withdrawal of forks or other handling attachments without damage to the pallet or load, - Manoeuvring a laden and unladen truck forward and in reverse in a narrowly confined area, - Performing both a left and a right 90º turn with a loaded truck. Fork tines are lowered when the truck is on route to required destination therefore reducing centre of gravity and tipping point of the load. FLT is fitted with roll cage protection. Driver wears seat belt whilst operating FLT. All loads moved by FLT are palletised to ensure secure loading onto the tines. Pallets are inspected for suitability before materials are loaded onto them. Defective pallets are removed from use. | 4 | 2 | 8 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FLT tipping over due to excess speed Driver could suffer personal injury or worse if the FLT were to tip over | 4 | 4 | 16 | Drivers are suitably trained/competent to operate the FLT they have been instructed to use. This training includes: - Competent use of controls, - Movement of the truck with forks or attachments in the correct travel position, laden and unladen. Speed limit signage, located throughout the premises and external areas, is followed by FLT drivers. Management/Supervisors monitor speeds and give verbal warnings if necessary. Repeat offenders are removed from using FLT and subject to company disciplinary procedures. FLT is fitted with roll cage protection. Driver wears seat belt whilst operating FLT. | 4 | 3 | 12 |  |

|  | FLT tipping over due to ground conditions of operating areas Driver could suffer personal injury or worse if the FLT were to tip over |  | 4 | 3 | 12 |  | Flooring/ground conditions subject to ongoing review (visual inspection) by FLT drivers and staff. Any changes in conditions e.g. cracking, pot holes etc. are reported immediately to the Company management team who arrange for suitable repair to be completed. Accumulations of loose material e.g. gravel etc. on the ground are cleared on a regular basis as they are particularly hazardous to small-wheeled lift trucks. The Company try to eliminate gradients along FLT routes where possible. Drivers never allow lift trucks to be driven up or down gradients that exceed the maximum specified by the manufacturer or authorised supplier. Drivers are suitably trained/competent to operate the FLT they have been instructed to use including never travelling or turning a FLT across a gradient. | 4 | 2 | 8 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Collision with other vehicles e.g. delivery/collection vehicles, company vehicles, staff cars Staff and/or other drivers could suffer personal injury or worse if their vehicle collided with a FLT moving through the external roadways | 4 | 3 | 12 | Clear, easy read warning signs are displayed in prominent positions to inform people that FLT’s are in operation in and around the premises and external areas. Informative notices instructing FLT drivers to sound horns at appropriate locations e.g. blind corners, entering buildings. FLT’s are fitted with audible warning devices e.g. automatic reversing bleepers, flashing warning beacons. Visibility aids are used where required e.g. convex mirrors at blind corners. High-visibility clothing is worn by pedestrians where the risks to their safety cannot be adequately controlled by other methods. FLT drivers wear high-visibility clothing at all times so they can be clearly seen when leaving the operating position of the truck. |  | 4 | 2 | 8 |  |
|  |  | Collision with buildings/access openings Driver could suffer personal injury or worse if the FLT collided with the building(s) or access openings | 4 | 3 | 12 | The Company make sure that roads and aisles have sufficient width and overhead clearance for the largest FLT to operate safely, whether loaded/unloaded, and if necessary, to allow other vehicles/loads to pass each other safely. One way system is used where possible, e.g. defined and differing entry and exit points. |  | 4 | 2 | 8 |  |

|  | FLT mechanical failure Driver could suffer personal injury if the mechanical operations of the FLT were to fail whilst in use | 3 | 3 | 9 | Daily checks are completed by FLT drivers before their shift starts. Any faults are reported to the Company supervisor who will arrange for suitable repair. Maintenance regime/contract in place and managed by the Company management team. This ensures that the FLT’s are subject to servicing and preventative maintenance on a regular basis. Thorough Examination completed by a competent person in line with statutory requirements (LOLER) FLT’s maintained by supplier (under contract) on a regular basis. | 3 | 2 | 6 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Parking and storage of fork truck Staff and/or 3rd parties may suffer injury due to tripping over the fork tines while it is parked up | 3 | 3 | 9 | The Company have established suitable storage/parking areas for FLT’s. These locations are away from areas where potential conflict or unauthorised access could occur. FLT drivers are trained on safe parking procedures (e.g. park truck away from pedestrian access and gangways, fork tines placed flat on the floor with a slight downward angle so ends of fork tines are flush with the floor) | 3 | 2 | 6 |  |
|  | Manual Handling Driver could suffer back injury when moving the tines along mast track, removing forks from mast track etc | 3 | 3 | 9 | Removal of fork tines is only carried out by the Company appointed service/maintenance contractor. Manual handling training/briefing is provided for FLT drivers to remind them of best practice when moving the fork tines. Tracks lubricated as part of the ongoing routine maintenance to assist with fork tine movement. Location lugs and catches are maintained and serviced which makes the operation easier for the FLT driver. Safety boots and gloves issued to all FLT drivers for use when moving the fork tines. | 3 | 2 | 6 |  |

|  | Use of non-integrated Working Platform Driver, person(s) on Working Platform, and/or pedestrians could suffer serious injury, even fatality if person(s) fall from height, items fall from height, person(s) on Working Platform make accidental contact with lifting mechanisms | 5 | 4 | 20 | Working platform is only used for non-routine maintenance tasks for which it is impractical to hire in purpose built access equipment, for tasks that would otherwise be carried out using less safe means of access such as ladders or because it is impractical to hire in purpose-designed people-lifting equipment due to the short duration and occasional nature of the task. Working Platform is of appropriate design – guard rails, toe boards, painted conspicuous colour, adequate means of securing to forks. FLT driver will remain at the controls of the vehicle at all times whilst the Working Platform is in use. FLT drivers suitably trained/briefed in the use of the Working Platform. Working Platform has an affixed identification plate and is compatible with FLT make and model. 6 monthly inspection of Working Platform and safety harnesses/lanyards completed by a competent person in accordance with statutory requirements. Pre-use checks by FLT driver to ensure equipment is safe, i.e. anchorage points, securely fixed to forks, safety harnesses/lanyards. Working area is cordoned off to prevent access and ensure safety of other non-essential personnel. | 5 | 2 | 10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Refuelling of FLT (diesel) Driver(s) and/or 3rd parties could suffer personal injury or worse if an incident were to occur whilst refuelling the FLT | 4 | 3 | 12 | Specific area established externally for the refuelling of vehicles. 3rd parties do not have access as the refuelling area is located away from general areas required by delivery/collection vehicles. Area kept clean, tidy and maintained as necessary by in-house staff. Diesel tank is bunded and access to the distribution pump restricted by lockable door. PPE worn by FLT drivers when refuelling i.e. non-absorbent gloves. | 4 | 2 | 8 |  |
|  | Checking battery fluid level Driver could suffer burns to skin and eyes due to splashes of battery acid when topping up battery | 3 | 3 | 9 | For unsealed cells only Driver ensures that the battery cell caps are unscrewed gently. The hydrometer is used correctly when checking cell specific gravity. Driver ensures that the cell caps are replaced properly after topping up. Wearing of relevant PPE including hand, eye, face and body protection. Eye wash available in First Aid kit. | 3 | 2 | 6 |  |
|  | Electrocution – connecting / disconnecting charging leads and/or battery from the FLT Driver could suffer burns to skin, eyes etc from short circuit / arcing. Danger of electric shock | 4 | 3 | 12 | Only completed by trained/competent personnel only who follow the manufacturer’s instructions. The driver confirms by visual inspection that the charger has been disconnected or isolated before connecting lead or leads. The driver ensures that the FLT is turned / locked off and can’t be accidentally switched back on (keys removed) The charging leads are connected correctly and in order – the earth leads last. | 4 | 1 | 4 |  |
|  | Changing of LPG LOT cylinder on forklift truck – potential release of gas/liquid, explosion Cold burns from direct contact, explosion of cylinder could affect any worker or visitor nearby at the time | 5 | 3 | 15 | LPG cylinder changed by authorised, trained personnel only. Manufacturer instructions followed when changing the cylinder. Safe system of work used which includes provision and use of appropriate PPE (gloves to prevent potential burns, eye protection) Potential sources of ignition controlled and combustible materials removed from the area. | 5 | 1 | 5 |  |

| ACTION ARISING FROM RISK ASSESSMENT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Risk Rating | Action Required: | Person (s) Responsible | Target Date | Date Completed |
| All | Various | FLT drivers to receive refresher training suitable for their knowledge and experience |  |  |  |

Risk Assessment Acknowledgement

The following TP operatives have read this risk assessment. 
Their signatures are confirmation that they have read and understood all which is within its contents.

| Name | Signature | Date | Name | Signature | Date |
| --- | --- | --- | --- | --- | --- |