| Area/Activity Assessed | Use of soldering iron | Date |  |
| --- | --- | --- | --- |
| Assessment Completed By |  | Person(s) Consulted |  |

| Persons Exposed | Employees |  | Contractor |  | Young Person |  | Expectant Mother |  | Visitors and/or Public |  | Trespassers |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency of Exposure | Continually |  | Hourly |  | Daily |  | Weekly |  | Monthly |  | Yearly |  |
| Duration of Exposure | Less than 1hr |  | 1-2 hrs |  | 3-4 hrs |  | 5-6 hrs |  | 7-8 hrs |  | More than 8 hrs |  |

| Probability (Prob) | 5= Very Likely, 4= Likely, 3= Quite Possible, 2= Possible, 1= Unlikely | Low | 0-8 | Low risk no action required |
| --- | --- | --- | --- | --- |
|  |  | Medium | 9-15 | Medium risk ensure adequate controls are in use |
| Severity (Sev) | 5= Catastrophic, 4= Major, 3= Moderate, 2= Minor, 1= Insignificant | High | 16-25 | High risk stop operation and implement adequate control measures |

|  | Task / Hazard |  | Initial |  |  |  | Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sev | Prob | Risk |  |  | Sev | Prob | Risk |  |
|  | Unauthorised use and operation of equipment. Operator could suffer personnel injury or cause injury to others as a result of using a tool or piece of equipment that they are unfamiliar with |  | 3 | 4 | 12 |  | Monitoring of work activities completed by nominated supervisor(s) who are located in the working areas throughout the day and ensure that good practice is followed. Only trained competent staff utilise equipment and a Competency Matrix has been established by the Company to ensure clarity of who is authorised to use named pieces of equipment. Specific safety, training, equipment authorisation and guidance documentation is readily available from the Company. Equipment/tools are securely stored when not in use. Unauthorised use of any tool or piece of equipment is deemed to be a direct violation of employee responsibility (section 7 HASAWA1974). Disciplinary action is used by the Company for any unauthorised use of tools and equipment. | 3 | 3 | 9 |  |
|  | High temperature The tip of the iron is at a temperature around 3000C and can cause burns to skin. Components being soldered will be at an elevated temperature for a short while after contact with the iron which can also cause minor burns and a sudden physical reaction to them which can cause other problems. Components should not be wet as contact with molten solder can cause spatter. |  | 3 | 4 | 12 |  | Soldering irons are used with a suitable station which has a suitable holster to rest the iron in when not immediately needed. The holster protects users from the hot iron. When using an iron there is a temptation to continue looking at the work when picking it from the holster and replacing it. This can lead to dropping it and picking it up by the wrong end which means users should make a conscious effort to look what they are doing with the iron at all times. Operatives shall avoid looking over the work too closely as this increases the risk of the hot iron touching the face and of inhaling fumes. Instead ensure the work area is well illuminated and use a magnifier if necessary. The magnifier will not only keep allow the operator to be further from the work but also put a barrier between them and the hot iron. | 3 | 1 | 3 |  |
|  | Fumes The soldering process gives off fumes from flux in the solder and minute quantities of vaporised solder. These can be harmful in high concentrations or over prolonged periods of exposure |  | 3 | 3 | 9 |  | For casual occasional soldering fumes will not present a significant hazard and no special precautions are necessary to avoid them. Where soldering is a frequent activity by one person or by several in a relatively confined area fumes are removed either by an effective extractor for the whole area or by local extraction and filtering at the station. Appropriate protective clothing is worn to prevent contamination of clothes from fumes (however, if the concentration of fumes is high enough for this to be of concern there will also be a significant risk from inhalation, and an extractor is employed) | 3 | 1 | 3 |  |
|  | Solid solder Solder is an alloy of soft metals which usually contains an integral resin flux. These will get onto and into the skin when the wire is being handled and can be harmful |  | 3 | 4 | 12 |  | Normal hygiene procedures are observed while soldering and immediately afterwards. No food or drink should is allowed at the soldering station and hands should be well washed before eating or performing other tasks. Disposable gloves are available. | 3 | 2 | 6 |  |
|  |  | Eye & musculo-skeletal strain Soldering for long periods can cause eye strain due to the close nature of the work and strain on the muscles and limbs from remaining in one position for a long time | 3 | 4 | 12 | The operative takes regular breaks from the work station to rest the body and eyes. Ensure that the body posture is changed and they are not performing similarly close work during the rest breaks. |  | 3 | 2 | 6 |  |
|  |  | Electrical faults Soldering irons need maintaining as with any other portable equipment but are susceptible to more strain than most portable equipment. In particular the cable joining the iron to the control station is susceptible to twisting and pulling strain and can be damaged by the hot iron. | 4 | 4 | 16 | Equipment is visually checked by user to spot early signs of damage or deterioration, including condition of lead and plug. Equipment is maintained, inspected and tested at periodic intervals. Damaged or faulty equipment is removed from use without delay and either: repaired by someone competent (i.e. with suitable training, skills and knowledge for the task to prevent injury to themselves or others); or disposed of to prevent its further use. Trailing leads are routed away from walkways or are covered to reduce possibility of accidental damage. |  | 4 | 1 | 4 |  |

|  | Housekeeping An untidy area, especially with paper and plastics on it, increases the risk of causing an accident with the hot iron | 3 | 4 | 12 | The work area is kept uncluttered with only necessary items close to the iron. Paper, such as that with the wiring diagram on it, and plastics, such as enclosures for circuit boards, are kept away from the immediate vicinity if possible. The iron is switched off when taking a rest break or after the work is finished. | 3 | 1 | 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| ACTION ARISING FROM RISK ASSESSMENT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Risk Rating | Action Required: | Person (s) Responsible | Target Date | Date Completed |
| 1 | Med | This risk assessment must be communicated to all staff involved in the use of the tool/equipment | Company Name supported by MBHS | 3 months from date of assessment |  |

Risk Assessment Acknowledgement

The following operatives have read this risk assessment. 
Their signatures are confirmation that they have read and understood all which is within its contents.

| Name | Signature | Date | Name | Signature | Date |
| --- | --- | --- | --- | --- | --- |