| Area/Activity Assessed | Work at Height | Date |  |
| --- | --- | --- | --- |
| Assessment Completed By |  | Person(s) Consulted |  |

| Persons Exposed | Employees |  | Contractors |  | Young Person |  | Expectant Mother |  | Visitors and/or Public |  | Trespassers |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency of Exposure | Continually |  | Hourly |  | Daily |  | Weekly |  | Monthly |  | Yearly |  |
| Duration of Exposure | Less than 1hr |  | 1-2 hrs |  | 3-4 hrs |  | 5-6 hrs |  | 7-8 hrs |  | More than 8 hrs |  |

| Probability (Prob) | 5= Very Likely, 4= Likely, 3= Quite Possible, 2= Possible, 1= Unlikely | Low | 0-8 | Low risk no action required |
| --- | --- | --- | --- | --- |
|  |  | Medium | 9-15 | Medium risk ensure adequate controls are in use |
| Severity (Sev) | 5= Catastrophic, 4= Major, 3= Moderate, 2= Minor, 1= Insignificant | High | 16-25 | High risk stop operation and implement adequate control measures |

This Risk Assessment and identified Control Measures has been established following consultation with Management, Supervisory and Operational staff

Staff acknowledgment as to their understanding of the control measures to be applied is included as the last page to this document

| Hazard |  | Initial |  |  | Existing Control Measures | Residual |  |  | Additional Controls |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |  |
|  | Falls from height Operatives could suffer severe personal injury or worse as a result of falling from height whilst working on behalf of the Company | 5 | 4 | 20 | Work at height is avoided wherever possible and alternative methods of completing the tasks required discussed with the project design team to establish if the issue can be “designed out” before construction work begins. Prior to any work at height being completed, an assessment of that work and the potential hazards are to be made either by the Company Project/Contract Manager or where the work is complex and involves more than one trade, the Principle Contractor in consultation with those trades and/or safety department/advisor. All operatives carrying out work at height included within the assessment process and potential hazards, safe working methods, access equipment requirements discussed and included within the assessment. All real or potential leading edges will be protected with suitable fall prevention methods e.g. handrails, mid-rails and low level edge protection. All fall prevention equipment will be suitable for use and constructed so that they are capable of withstanding a foreseeable load falling against it. | 5 | 2 | 10 |  |  |

| Hazard |  | Initial |  |  | Existing Control Measures | Residual |  |  | Additional Controls |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |  |
|  | Falling materials Operatives and 3rd parties working in close proximity to work at height could suffer severe personal injury or worse if they were struck by falling materials | 5 | 4 | 20 | Physical barriers are used to create a designated “authorised person” area which will prevent access by other site users to the area directly below where work at height is being completed. Edge protection at low level is in place on any leading so as to prevent accidental loss of tools, equipment etc. over the work platform to the area below. Storage of materials, equipment, waste etc. on work platforms that are deemed to be at height is minimised as far as reasonably practical and only the tools/equipment etc. required for the task in hand is deemed necessary. Where necessary, operatives will utilise tool belts with lanyards attached to tools to prevent them from being dropped. | 5 | 2 | 10 |  |  |
|  | Falls from height – from steps/hop-up Operatives could suffer severe personal injury or worse as a result of falling from height whilst using step ladders or hop-ups | 4 | 3 | 12 | Steps and hop-ups are subject to visual inspection by the operative before use and weekly by the appointed Company supervisor who records any issues or conditions that are observed. Equipment used for working at height is fit for purpose and is selected following informal assessments completed by the Company Management Team during pre-start meetings and surveys. Step ladders used on site are class 1 (BSEN131), suitable for industrial use and for light works of short duration. Alternate means of accessing works at height is considered before step ladders are deemed necessary. Hop-ups are fit for purpose and made by a manufacturer (not by adapting other equipment e.g. boxes/crates etc.). 600x600 hop-up platforms are the preferred option, however there may be requirement for 600x300 platforms where space is limited. Operatives trained in safe use of steps and ongoing toolbox talks used as a means of reminding them of information. Steps and hop-ups are only used on flat, solid ground and only for short duration | 4 | 2 | 8 |  |  |

| Hazard |  | Initial |  |  | Existing Control Measures | Residual |  |  | Additional Controls |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |  |
|  | Fall from height of operatives working from scaffold Operatives – could suffer severe personal injury or worse if they were to fall from height working from the scaffold | 5 | 4 | 20 | Scaffold erected by suitably competent accredited scaffold contractor appointed by the Principle Contractor who ensures that the scaffold is subject to suitable inspection regime by Client appointed competent person e.g. every 7 days, after alteration or adverse weather conditions as a minimum Operatives follow site rules regarding access and safe use of scaffold to ensure that any restrictions that may have been imposed by the Principle Contractor/scaffold contractor are followed. Operatives ensure that they do not lean over or past any protective guard rail. The Company supervisor liaises with the site management team if any alteration or adaption is required for the Company work activities. The scaffold will/must not be altered by Company operatives – issues or problems are reported to the site management team Operatives are reminded during the pre-start task briefing that the dropping of materials/equipment/waste is strictly prohibited by Company and is not tolerated under any circumstances. Weather conditions will be monitored when working at heights and operations will be curtailed when the stability of any access equipment is under question. | 5 | 2 | 10 |  |  |
|  | Fall from height of operatives working from Mobile Elevated Work Platform (MEWP) Operatives – could suffer severe personal injury or worse if they were to fall from height working from a MEWP | 5 | 4 | 20 | Most appropriate MEWP selected at initial site review ensuring that the existing environment and hazards are taken into consideration e.g. condition of ground surface, slopes, changes in elevation, other vehicle/pedesrtians using or within the locale. Only trained competent operatives will use/operate the selected MEWP in accordance with relevant standards and manufacturer’s instructions Specification for MEWP will take into account the site ground conditions expected, height restrictions and obstructions Use of MEWP will be on flat ground and suitably secured to prevent overturning. MEWP will be manoeuvred at slow speed by operative After alteration or adverse weather conditions, MEWP must be inspected by a competent person every seven days if they remain in place, and the results recorded. | 5 | 2 | 10 |  |  |

| Hazard |  | Initial |  |  | Existing Control Measures | Residual |  |  | Additional Controls |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |  |
|  | Fall from height of operatives working from mobile tower scaffold Operatives – could suffer severe personal injury or worse if they were to fall from height working from the mobile tower scaffold | 5 | 4 | 16 | Only trained competent operatives will erect, modify or dismantle mobile scaffold towers in accordance with relevant standards and manufactures instructions. A suitably competent person (PASMA trained) will check the tower prior use to confirm it has been erected correctly. Specification for a mobile scaffold tower will take into account the site ground conditions expected, height restrictions and obstructions Ladder access to the working platforms of the tower will be internal and fixed to the narrowest side Maximum height to base ration will not exceed 3:5:1 inside use and 3:1 external use without ties All mobile scaffold tower work platforms will be fully boarded with toe-boards and handrail and intermediate guard rails. Personnel and materials will be removed from working platforms before the mobile tower scaffold is moved. An outrigger will be installed at each corner following the manufacturer guidance. These outriggers will provide additional stability and rigidity to the mobile tower scaffold. After alteration or adverse weather conditions, towers must be inspected by a competent person Towers over 2m in height must be inspected every seven days if they remain in place, and the results recorded | 5 | 2 | 10 |  |  |

| ACTION ARISING FROM RISK ASSESSMENT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Risk Rating | Action Required: | Person (s) Responsible | Target Date | Date Completed |
|  | All | This risk assessment must be communicated to all those working at height on behalf of the Company | Company Directors | 3 months from date of assessment |  |
|  | All | This risk assessment must be subject to ongoing review, annually as a minimum, so that it remains suitable and sufficient in relation to the general processes for working at height. Specific risk assessments MUST be established for those work activities that pose a significant issue that hasn’t been captured by the general assessment details above. | Company Directors | Ongoing |  |

Acknowledgement

The following members of staff have read this risk assessment.

Their signatures are confirmation that they have read and understood all which is within its contents.

| Name | Signature | Date | Name | Signature | Date |
| --- | --- | --- | --- | --- | --- |