| Area/Activity Assessed | General Premises Jack Arnold UK | Date | 27th May 2025 |
| --- | --- | --- | --- |
| Assessment Completed By | TSA and JA MB Health and Safety Ltd | Person(s) Consulted | Lee Marden (Operations Manager) |

| Persons Exposed | Employees |  | Contractors |  | Young Person |  | Expectant Mother |  | Visitors and/or Public |  | Trespassers |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency of Exposure | Continually |  | Hourly |  | Daily |  | Weekly |  | Monthly |  | Yearly |  |
| Duration of Exposure | Less than 1hr |  | 1-2 hrs |  | 3-4 hrs |  | 5-6 hrs |  | 7-8 hrs |  | More than 8 hrs |  |

| Probability (Prob) | 5= Very Likely, 4= Likely, 3= Quite Possible, 2= Possible, 1= Unlikely | Low | 0-8 | Low risk no action required |
| --- | --- | --- | --- | --- |
|  |  | Medium | 9-15 | Medium risk ensure adequate controls are in use |
| Severity (Sev) | 5= Catastrophic, 4= Major, 3= Moderate, 2= Minor, 1= Insignificant | High | 16-25 | High risk stop operation and implement adequate control measures |

This Risk Assessment and identified Control Measures has been established following consultation with Management, Supervisory and Operational staff.

Staff acknowledgment as to their understanding of the control measures to be applied is included as the last page to this document.

| Hazard |  | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
|  | Vehicle movements leading to collision between vehicles and people Staff could suffer severe personal injury or worse as a result of collisions with delivery/collection vehicles | 4 | 3 | 12 | - Designated access route to the Company premises from the main highway. - Designated pedestrian-only walking areas marked out. - There is a designated loading/Unloading area at the rear of the building - The Company has established suitable car park facilities for employees and spaces for visitors externally to the main building. - Pedestrian access points are segregated from vehicle routes to reduce the risk of interaction. - Sufficient lighting in place externally to ensure that suitable illumination of access routes is available, even in low light conditions | 4 | 2 | 8 |  |
|  | Unauthorised access to the premises Staff could suffer severe personal injury or worse as a result of trespassers or unauthorised personnel gaining access to working areas | 3 | 3 | 9 | - All entrance doors and shutters are fitted with suitable security locks and are secured when not in use. - Key access is controlled by the senior management team to restrict unauthorised entry - All visitors are to be accompanied by staff at all times while on the premises - Side access gates are to be kept locked when the premises are not in use. | 3 | 2 | 6 |  |

| Hazard |  | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Adverse weather creating hazardous access/egress conditions Staff and 3rd party personnel could suffer personal injury due to weather conditions creating a hazardous condition | 3 | 4 | 12 | - The Company ensures all roadways and pedestrian access routes that are within its responsibility are cleared of snow/ice. - A ready supply of “salt/grit” is to be readily available to ensure that slippery access routes are made as safe as far as reasonably practicable. - All vehicle drivers, whether employees arriving for work, commercial vehicle drivers delivering/collecting and visitors, are required to take extra caution when road conditions may be hazardous as required by the Highway Code & Road Traffic Act | 3 | 2 | 6 |  |
|  | Fire / explosion Staff and other building users could suffer from smoke inhalation, burns or worse if trapped in the premises | 5 | 3 | 15 | - The Company, as the “responsible person,” ensures a specific fire risk assessment is completed in line with legislative requirements (Regulatory Reform (Fire Safety) Order 2005). - The assessment considers the provision of preventative measures, means of raising the alarm, staff training, and provision of firefighting equipment. - Gas pipes (LPG) are fixed to avoid accidental collision and away from main access routes used by vehicles, where possible - Smoking prohibited in the building in line with current legislation | 5 | 2 | 10 |  |
|  | Housekeeping Staff and others could suffer injury as a result of poor housekeeping creating hazards in the working area | 3 | 3 | 9 | - Welfare areas are subject to a daily cleaning regime by employees, as well as a twice weekly clean by a cleaning contractor. - Staff are aware of their responsibility regarding the need for a high standard of housekeeping in their working area. - The factory staff routinely clean the workspace around their designated workspace to minimise contamination and maintain safety. - The disposal of any Swarf and waste material is done responsibly and in accordance with the directions provided by the manufacturer and according to local authority regulations | 3 | 2 | 6 |  |
|  | Slips, trips, and falls Staff and 3rd party personnel could suffer personal injury as a result of falling as a result of the premises flooring being in poor condition | 4 | 3 | 12 | - The internal facilities have been modernised with good standard of flooring throughout the area used by staff and visitors. All access is on the ground floor. - Stairs throughout the premises are in good condition and have appropriate handrails and metal staircases have nonslip treads. - All employees are aware of the need for good housekeeping throughout the premises. This ensures that all access routes, walkways, and general floor areas are kept free from obstructions and potential trip hazards. - All access routes, walkways, and general floor areas are suitably illuminated using an appropriate level of lighting. - Appointed cleaning contractor maintains the office and welfare areas on a twice-weekly basis. | 4 | 2 | 8 |  |

| Hazard |  | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Slips, trips, and falls Cont… Staff and 3rd party personnel could suffer personal injury as a result of falling as a result of the premises flooring being in poor condition | 4 | 3 | 12 | - Floor surfaces of the premises areas are to be subject to ongoing visual inspections by all employees throughout the working day - Designated walkways are to be kept clear and free from obstructions. Storage is not permitted in any designated walkway. - On-going supervision is completed by a nominated Foreman who ensures employees are adhering to safe working practices. | 4 | 2 | 8 |  |
|  | Materials/equipment storage using shelving and racking within the warehouse, leading to potential falling objects Staff and 3rd parties could suffer severe personal injury or worse as a result materials falling onto them | 4 | 3 | 12 | - All materials are to be stored appropriately on pallets, racking and shelving. - Storage areas are maintained by a nominated Foreman who ensures that the racking system is not overloaded. - Materials required for fabrication processes are stored appropriately in designated areas. Only materials required for fabrication order completion are located in the machine/factory areas. - Employees are responsible for ensuring that any/all materials in their defined area(s) are stored in a manner that does not create a hazard for anyone. - The wearing of protective footwear (PPE) is mandatory in designated areas. - On-going supervision is completed by a nominated Foreman who ensures employees are adhering to safe working practices. - Racking provided is of suitable density and strength regarding product stored | 4 | 2 | 8 |  |
|  | Electricity (fixed installation) Staff and 3rd parties could suffer severe personal injury or worse if they were to come into contact with an unsafe electrical installation | 5 | 3 | 15 | - The fixed electrical installation is tested and inspected, at appropriate intervals, by a competent person (e.g. Industrial: 3 yearly, Commercial: 5 yearly). - A copy of the last electrical condition report is to be readily available from Jack Arnold management team. - Fuses, Miniature Circuit Breaker (MCBs) and isolation switches are labelled with the circuits that they isolate and control. - Switch rooms are kept locked and inaccessible to unauthorised personnel. - Residual current devices (RCD’s) are incorporated into the fixed electrical system. - Unobstructed access is provided to all electrical isolators and switchgear. | 5 | 1 | 5 |  |

| Hazard |  | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Lighting Poor lighting could result in higher levels of work error, eyestrain, and accidents | 3 | 3 | 9 | - Lighting is sufficient to enable people to work, use facilities without experiencing eyestrain, and safely move from place to place. - Artificial lighting is provided at individual workstations and at places of particular risk such as pedestrian access routes. - Stairs are well lit, and the lighting does not cast shadows over the main part of the treads. - Outdoor traffic routes used by pedestrians are adequately lit after dark. - Lights and light fittings have been positioned and maintained, so that they avoid annoying glare and do not cause a hazard (e.g. electrical, fire or collision). - Lights and windows are cleaned, repaired or replaced, as necessary, before the level of lighting becomes insufficient. personnel avoid obscuring light by placing items in front of lights and windows (e.g. stacked goods). - Light switches are positioned so that they may be found and used easily and without risk. | 3 | 2 | 6 |  |
|  | Thermal comfort Staff and 3rd parties could suffer general discomfort or ill health as a result of adverse temperature in indoor workplaces | 3 | 3 | 9 | - The temperature inside the office areas provides reasonable comfort without the need for special clothing, due to the installed heating system. - The temperature inside the factory is controlled with localised mobile heating units (Halogen Infra-red heater Devil 370PD) - In extremely hot weather, fans and increased ventilation are used instead of local cooling. - Suitable protective clothing and rest facilities are provided in instances where local heating or cooling fails to give reasonable comfort. - Task rotation ensures the amount of time individual workers are exposed to uncomfortable temperatures is limited. | 3 | 2 | 6 |  |
|  | Space (room dimensions) Staff and 3rd parties could suffer discomfort as a consequence of restriction on movement around the workroom. A congested and restrictive workspace may result in higher levels of accidents | 3 | 3 | 9 | - Workroom has enough free space to allow people easy access to and from workstations, to move within the room with ease and does not restrict movements while performing their work. - Workroom does not exceed the recommended minimum space requirement of one person per 11m³ (ref. l24). - Where space is limited, careful planning of the workplace is considered. - Obstructions are clearly marked (e.g. low beams). | 3 | 2 | 6 |  |

| Hazard |  | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Storage of hazardous substances (cleaning chemicals e.g., bleach) Inadequate storage arrangements could lead to potential fire/explosion risk or ill health as a result of coming into contact with substances via touching and inhalation | 3 | 3 | 9 | - CoSHH inventory, material Safety Data Sheets (MSDS) and corresponding CoSHH risk assessments completed (If/where required) for those substances identified as being hazardous. - Access to the chemical store is restricted to authorised personnel. - Hazardous storage is to be provided with sufficient space, well organised, well lit (intrinsically safe) and well ventilated. - Products are to be stored securely in a cool and dry location, capable of capturing spills using bunds, stock control procedures minimise the number of hazardous substances stored. - Incompatible materials are to be stored safely, areas are defined, and clear signs are displayed. - Suitable and sufficient warning signs displayed where required. - Containers are stored so their labels face forward. - Ignition sources are adequately controlled or eliminated within the hazardous storage areas - Suitable fire extinguishing equipment is provided locally. | 3 | 2 | 6 |  |
| - 10 | Charging of electric vehicles. All site personnel are potentially at risk from electric car fires and could suffer personal harm from inhalation of smoke/gasses and/or direct contact with flames/explosion. | 4 | 3 | 12 | - Car charging points are installed by an appointed competent contractor, installation certificates are to be kept on file and are incorporated in the periodic inspection and test requirement. - Only use the correct charging lead as outlined by the vehicles manufacturer. - The charging of electric vehicle is also captured in the fire risk assessment. - All electric vehicle owners ensure maintenance is undertaken in line with the manufacturer’s instructions. - If an electric car enters in thermal runaway (Release of gasses and/or explosions) never try to enter the car, adhere to the emergency procedures and make your way to the fire assembly point and raise the alarm. - There is sufficient space for vehicles to parked safely in the designated charging area, and for a connection to be made to the charging equipment, charging equipment and cables do not interfere with any access or emergency egress routes. | 4 | 2 | 8 |  |
|  | Forklift Truck (FLT) movements in warehouse/yard areas leading to collision/conflict with people and objects. Staff and 3rd parties could suffer severe personal injury or worse because of collisions with delivery/collection vehicles | 4 | 4 | 16 | - FLT operators are competent, adequately trained and authorised. - All visitors sign a form receiving safety information and are required to remain within the designated areas when viewing slabs in the warehouse area (Hi-vis vests required to worn), all visitors are to be escorted. - FLT drivers are aware that they must drive at slow speed when in the factory areas, - Illumination levels within the Factory are adequate which will reduce the risk of collision with any people and/or objects. | 4 | 3 | 12 |  |

| Hazard |  | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Storage of LPG cylinders for FLT’s and Argon for welding activities Inadequate storage arrangements could lead to potential fire/explosion risk or ill health as a result of coming into contact with gas via touching and inhalation | 5 | 3 | 15 | - Cylinders are to be stored in a dedicated open-air store. - The storage area is to be kept free from weeds, long grass and accumulations of readily combustible material at all times. - Appropriate warning notice is to be installed to the store. - Cylinders are to be stored with their valves uppermost. This ensures that if there is a leak from the valve area, vapour and not liquid will escape. - The position chosen for storage is level and concreted, to provide a load-bearing surface. - There are to be no openings into buildings within 2m of the LPG storage area. - The storage area is not accessible to the general public or other unauthorised persons. - Suitable barriers are used to prevent unauthorised access and accidental damage. - A 9kg Dry powder fire extinguisher is to be provided in the vicinity of the cylinder storage, and it is serviced regularly. | 5 | 3 | 15 |  |
|  | Exposure to work-based noise. Staff and 3rd parties could suffer work-based noise induced hearing loss if prolonged exposure to noisy environments | 3 | 4 | 12 | - Noise assessment completed using simple non-calibrated sound level meter (SSLM) to ascertain noise levels in areas of concern | 3 | 4 | 12 |  |
|  | Power operated doors/gates. Staff and 3rd parties could suffer severe personnel injury if struck by moving door/gate | 4 | 3 | 12 | - Upward opening doors (e.g. roller shutters) fitted with a counterbalance or ratchet mechanism device to prevent them falling back in a manner likely to cause injury. - ‘Hold-to-operate’ control, when released, causes the door or gate to stop or reopen immediately. The operator has a clear view of the door throughout its movement. - Readily identifiable and accessible control switch or device – so that they can be stopped quickly in an emergency. | 4 | 2 | 8 |  |

| ACTION ARISING FROM RISK ASSESSMENT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Risk Rating | Action Required: | Person (s) Responsible | Target Date | Date Completed |
| All | Medium | Ensure that all staff involved in tasks associated with this risk assessment read and sign this risk assessment | Jack Arnold Management | Ongoing |  |
| 1 | High | Personnel are required to wear high-visibility clothing in designated areas where there is a risk of interaction with forklift trucks, in accordance with HSG136 A guide to workplace transport safety – segregation of pedestrians. | Jack Arnold Management | Ongoing |  |
| 2 | Medium | Establish procedures for the control and management of visitors, including mandatory sign-in protocols and clearly defined standards of visitor conduct following site regulations. | Jack Arnold Management | 1 Month |  |
| 3 | Medium | Confirmation is required to ensure the ongoing monitoring and enforcement of compliance with winter weather safety measures for vehicle and pedestrian routes. | Jack Arnold Management | 1 Month |  |
| 4 | High | Fire risk assessment to be reviewed along with actions that have been raised, a review of staff training, statutory maintenance and testing by a competent person to be implemented and establish a recording of in-house testing of fire provisions, preferably within the fire logbook. | Jack Arnold Management | Ongoing |  |
| 5 | High | Waste generated during machine operation must be vacuumed rather than dry swept and disposed of in designated waste bins following site waste management procedures Vacuuming (hoovering) dry waste is recommended over sweeping because it reduces airborne dust, improving respiratory health and preventing contamination of the workplace | Jack Arnold Management | Ongoing |  |
| 5 | Medium | Materials stored or placed improperly are encroaching into designated safe working zones, causing restricted access and preventing the effective use of welding screens, thereby increasing the risk of accidents and exposure to welding hazards Recommend that a review take place for storage and placement of materials to ensure all safe working areas remain unobstructed. | Jack Arnold Management | 1 Month |  |
| 6 | Medium | Trailing electrical cables and improperly stored tools present significant tripping risks, potentially causing injury to staff and visitors. This can be reduced by implementing effective cable management, including cable covers, trays, or securing cables along walls or overhead, wherever possible, to eliminate tripping hazards. Staff are to ensure all tools are stored properly in designated storage areas immediately after use to maintain clear and safe working spaces | Jack Arnold Management | 1 Month |  |
| 8 | Medium | To strengthen awareness and compliance with the requirement for protective footwear in designated areas, additional clear and visible signage must be installed at all relevant entry points and high-risk zones within the workplace. Ensure that all signage clearly states: “Protective Footwear Must Be Worn.” | Jack Arnold Management | 1 Month |  |
| 8 | Medium | Regular racking inspections must be carried out by a competent person. Checks must confirm the racking is safe and free from damage or overloading, with records kept either on-site or digitally. Any issues must be addressed immediately in line with the Provision and Use of Work Equipment Regulations (PUWER) 1998, the Workplace (Health, Safety and Welfare) Regulations 1992 , and the Health and Safety at Work Act. 1974 | Jack Arnold Management | 1 Month |  |
| 9 | Medium | Electrical Inspection Condition Report to be reviewed by MB Health and Safety, no documentation available at the time of visit – Date on distribution board September 2024. | Jack Arnold Management | 1 Month |  |
| 12 | Medium | Workstations must be reviewed to ensure low beams or structural frames do not encroach on the working environment or introduce additional safety hazards. Workspaces should be organised to allow operators to complete tasks safely, with sufficient clearance and without unnecessary obstruction or risk | Jack Arnold Management | 1 Month |  |
| 13 | Medium | A current COSHH inventory and all relevant Material Safety Data Sheets (MSDS) must be readily available, accessible to all relevant personnel, and kept up to date to ensure compliance and safe handling of hazardous substances in accordance with the Control of Substances Hazardous to Health (COSHH) Regulations 2002. | Jack Arnold Management | 1 Month |  |
| 13 | Medium | A COSHH cupboard must be purchased and installed in a clearly identified, appropriate location that is away from fire evacuation routes, stairways, and high-traffic areas, in order to securely store hazardous substances | Jack Arnold Management | 1 Month |  |
| 14 | Medium | No evidence was available to show that the vehicle charging station is subject to service and testing under the manufacturer's service agreement. Contact should be made with the manufacturer for further guidance on any specific requirements that could impact fire safety. | Jack Arnold Management | 1 Month |  |
| 15 | Medium | The Company should ensure that all FLT operators are subject to ongoing refresher training to maintain their competency. A review should be undertaken of all FLT operators and a plan of refresher training requirements established. | Jack Arnold Management | Ongoing |  |
| 16 | High | During visit, it was identified that a significant quantity of LPG and Argon cylinders were not stored in accordance with required safety protocols (DSEAR, HSG51, COSHH). This presents a serious risk of fire, explosion, and injury. Immediate action is required to ensure all LPG and Argon cylinders are relocated to a designated open-air storage area. | Jack Arnold Management | 1 Month |  |
| 17 | Medium | Noise assessment completed using a simple non-calibrated sound level meter (SSLM) to ascertain noise levels in areas of concern, showed noise levels at between 85-90 decibels from approximately 10-20 meters away from workstations. Recommend that a comprehensive noise survey be carried out by a qualified specialist technician to accurately evaluate the noise exposure and determine the appropriate hearing protection requirements in accordance with the Control of Noise at Work Regulations 2005 | Jack Arnold Management | 3 Months |  |
| 18 | Medium | Additional information required on the safety features of the roller shutters that are installed leading to the factory areas from the courtyard. | Jack Arnold Management | 1 Month |  |

Acknowledgement

The following members of staff have read this risk assessment.

Their signatures are confirmation that they have read and understood all which is within its contents.

| Name | Signature | Date | Name | Signature | Date |
| --- | --- | --- | --- | --- | --- |