| Area/Activity Assessed | Operating MACC horizontal band saw (Special 280) |  |  | Date | 30th October 2025 |
| --- | --- | --- | --- | --- | --- |
| Assessed by | Tony Smith-Allen | Assessed with | Lee Marden (Operations Manager) & Ilyan Karashtranov (Factory Manager) |  |  |

| Persons Exposed | Employees |  | Contractor |  | Young Person |  | Expectant Mother |  | Visitors and/or Public |  | Trespassers |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency of Exposure | Continually |  | Hourly |  | Daily |  | Weekly |  | Monthly |  | Yearly |  |
| Duration of Exposure | Less than 1hr |  | 1-2 hrs |  | 3-4 hrs |  | 5-6 hrs |  | 7-8 hrs |  | More than 8 hrs |  |

| Probability (Prob) | 5=Very Likely, 4= Likely, 3= Quite Possible, 2= Possible, 1= Unlikely |  | 0-8 - Low risk No Action Required. 9-15 - Medium risk Ensure adequate controls are in use. 16-25 - High Risk Stop operation and implement adequate control measures |
| --- | --- | --- | --- |
| Severity (Sev) | 5=Catastrophic, 4=Major, 3=Moderate, 2=Minor, 1=Insignificant |  |  |

This Risk Assessment & Control Measures has been established following consultation with Management, Supervisory and Operational staff.

Staff acknowledgment as to their understanding of the content and the control measures to be applied is included as the last page to this document

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 1 | Manual Handling Staff - could suffer from back pain if objects carried/moved incorrectly, there is repetition of the process and/or speed of the machine dictates production | 3 | 4 | 12 | - Profile bar lengths are located in close proximity to the machine which reduces distance of travel required to carry. Metal being cut is accessible via the main shutter entrance to the front of the building where the machine is located. - Profile bar lengths are moved into position onto the machine bed by the operator(s), ensuring that their own capabilities are not compromised. Other staff are in close proximity to provide assistance with lifting if required. - The machine bed is at a height that reduces the need for stretching, over-reaching etc to aid ease of loading. - Manual handling guidance provided within the induction process and informally on an ongoing basis by the Factory Manager (IK) & Company Management - Monitoring of work activities completed by the management team who ensures that manufacturing and fabrication processes include good manual handling practice. | 3 | 2 | 6 |  |
| 2 | Machine failure leading to potential impact on Staff e.g. electrocution Staff and others - could suffer electrical shock, burns or personal injury if equipment is faulty | 4 | 3 | 12 | - Pre-use maintenance check completed by the operator to ensure that the machine visually appears in good order and ready to use. - Concerns are raised with the Factory Manager (IK) and/or Company Management who contacts external specialists for solutions. - Machines are subject to routine maintenance by appointed external specialists who visit on a regular basis. - Where required, machines that are awaiting repair or are damaged, are taken out of use and isolated at power sources to prevent use. - Machines are never used until issues or faults have been resolved appropriately. | 4 | 2 | 8 |  |

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 3 | Entrapment/Entanglement leading to cuts, abrasions, crush injury Staff could suffer personnel injury if they are were to become entangled in the machine whilst in operation | 4 | 4 | 16 | - Metal work staff completes daily checks of all machinery guarding to ensure that it is in place, working correctly and the machine is ready for use. - Metal profile is lifted onto the machine bed by the operator and then secured in position by clamps as part of the machine operation. - The cutting zones is partially protected by cover guards that enclose the blade and limits the exposed blade to the cutting area which protects the operator from any potential ejection of materials. - The machine operation requires the operator to de-press the trigger on the pull-down handle that allows the blade to contact the material. The trigger “fails to safe” and the machine stops if the trigger is released. - The trigger automatically instigates the flow of coolant along the blade when cutting commences. The blade operates at relatively slow speed and therefore the exposure to any airborne particles or mists is not significant. - Emergency stop button located on control panel and readily accessible. - The manufacturer has identified that eye, hearing protection is required along with anti-cut gloves when handling the profile. - All fabrication staff have been trained both in-house and by the machine supplier. Records of training are held by the Company. - Manufacturer machinery guide is located in the office and can be referred to at any time if any concerns or issues are raised. | 4 | 2 | 8 |  |
| 4 | Unauthorised Use Staff could suffer personnel injury if they are using a machine that they are unfamiliar with | 3 | 3 | 9 | - Monitoring of work activities completed by the Company Management who ensure that good practice is followed. - The management oversee and manage the use of the machine on an ongoing basis to ensure that it is used correctly and to raise concerns or issues if others appear to be using it incorrectly. - Specific safety, training, machine authorisation and guidance documentation is readily available in the office. - Unauthorised use of any machine is deemed to be a direct violation of employee responsibility (section 7 HASAWA1974). Disciplinary action is used by the Company for any unauthorised use of machines. | 3 | 2 | 6 |  |

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 5 | Contact with substances/materials Staff could suffer occupational dermatitis, Asthma and other ill health effects caused by exposure to substances | 3 | 4 | 12 | - Material being cut (steel) generates a swarf that is heavier than air and therefore limits the level of any airborne contaminants created by the various routing and cutting process to within the machine bed and not a significant issue. - Products/substances used during the machining process are limited to those required by the machine itself e.g. coolant which are located within a sump that feeds the machine blade when it is in motion, which is away from the operator. - Products used during fabrication process are limited and are stored in a designated area. Only required quantities of substances (cleaners, sealants) are located within the area. - Safety Data Sheets (SDS) for any hazardous substances used are held in the office and are available for review at any time. | 3 | 4 | 12 |  |
| 6 | Housekeeping Staff and others could suffer injury as a result of poor housekeeping creating hazards in the working area | 3 | 4 | 12 | - All staff are aware of their responsibility regarding the need for a high standard of housekeeping in their working area. - Waste generated is swept from the floor during the day and placed within specific bins in readiness for disposal and subsequent recycling if possible. Operations waste disposed of in bins located throughout the premises for disposal. - Swarf that may be generated and deposited to the rear of the machine to be included in all clean-up processes so that it is not allowed to build-up to significant amount | 3 | 2 | 6 |  |
| 7 | Exposure to work based noise Staff and others could suffer from noise induced hearing loss over a period of time if exposed to high levels of work based noise | 3 | 3 | 9 | - Noise level readings vary dependent on size, structure of the profile being cut. - Review of noise levels completed using an uncalibrated noise “app” to ascertain indicative noise levels in areas of concern. - Machine operates at Operations levels of between 1st and 2nd action level (80 – 85dba) but with additional noise provided by other machines in close proximity. Machine operations are automatic, for short duration and not constant. - Some noise protection is provided within the machine itself, but the manufacturer advises that the wearing of hearing protection is mandatory whilst machine is in use. - Hearing protection, in-ear buds & over-ear defenders are readily available via the dispenser within the factory or from the office. All PPE, including hearing protection is provided to all fabrication staff free of charge. | 3 | 2 | 6 |  |

| ACTION ARISING FROM RISK ASSESSMENT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Risk Rating | Action Required: | Person (s) Responsible | Target Date | Date Completed |
| 1 | Low | In-house training/briefing on manual handling, fire safety awareness and this risk assessment to be provided to all fabrication staff. Monthly H&S briefings/toolbox talks on relevant subject(s) to be issued to the fabrication staff | Jack Arnold Management | Annually Monthly |  |
| 2 | Low | Records of all machine maintenance, inspections and repairs to be held as an ongoing reference of the works completed on each machine. Information should be kept under review to ascertain if any trends occur such as breakdowns, frequent repairs of similar nature, replacement components etc. | Jack Arnold Management | Ongoing | N/A |
| 3 | Low | Factory Foreman to provide ongoing monitoring of work activities that all staff are following agreed safe methods of work and processes. | Jack Arnold Management | Ongoing | N/A |
| 5 | Medium | Manufacturers/suppliers of products used by the Company to provide Safety Data Sheets (SDS). Latest SDS to be reviewed and CoSHH assessments to be reviewed annually (minimum) and updated as necessary. | Jack Arnold Management Assisted by MBHS Ltd | Annually as a minimum |  |
| 5 | Medium | The machine to be subject to thorough cleaning, including the coolant/water sump. HSE guidance document MW3 to be used as a method of cleaning. Routine testing of the metal working fluids (MWF) to be completed by management team. Frequency to be confirmed once appropriate testing kit is purchased (kit manufacturer and HSE guidance to be used as a basis for frequency of testing) | Jack Arnold Management | Ongoing |  |
| 7 | Low | Review sound/noise levels at the operator position within the factory as it is more likely, due to the small size of the area with a cumulative level of noise, needs to be addressed. | Jack Arnold Management | Annually as a minimum |  |
|  | Various | Ensure that all signage relevant to the machine (mandatory PPE, Hazard warning, Prohibitive signs) is clearly visible and displayed in a prominent position to act as an ongoing reminder to the operator | Jack Arnold Management | Ongoing | N/A |
|  | Various | Ensure that PPE identified by the machine manufacturer (identified by the blue mandatory signage on the machine) is worn by the operator as directed. | Jack Arnold Management | Ongoing | N/A |
|  | Various | Company management to continue Weekly routine & inspections of the premises (and works being completed) so that a record of any actions or required improvements can be held. | Jack Arnold Management | Weekly |  |

Risk Assessment Acknowledgement

The following members of staff have read this risk assessment. 
Their signatures are confirmation that they have read and understood all which is within its contents.

| Name | Signature | Date | Name | Signature | Date |
| --- | --- | --- | --- | --- | --- |