| Area/Activity Assessed | Use of Guimadira Press & Shear |  |  | Date | 30th October 2025 |
| --- | --- | --- | --- | --- | --- |
| Assessed by | Tony Smith-Allen (MBHS) | Assessed with | Lee Marden (Operations Manager) & Ilyan Karashtranov (Factory Manager) |  |  |  |

| Persons Exposed | Employees |  | Contractor |  | Young Person |  | Expectant Mother |  | Visitors and/or Public |  | Trespassers |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency of Exposure | Continually |  | Hourly |  | Daily |  | Weekly |  | Monthly |  | Yearly |  |
| Duration of Exposure | Less than 1hr |  | 1-2 hrs |  | 3-4 hrs |  | 5-6 hrs |  | 7-8 hrs |  | More than 8 hrs |  |

| Probability (Prob) | 5=Very Likely, 4= Likely, 3= Quite Possible, 2= Possible, 1= Unlikely |  | 0-8 - Low risk No Action Required. 9-15 - Medium risk Ensure adequate controls are in use. 16-25 - High Risk Stop operation and implement adequate control measures |
| --- | --- | --- | --- |
| Severity (Sev) | 5=Catastrophic, 4=Major, 3=Moderate, 2=Minor, 1=Insignificant |  |  |

This Risk Assessment & Control Measures has been established following consultation with Management, Supervisory and Operational staff.

Staff acknowledgment as to their understanding of the content and the control measures to be applied is included as the last page to this document.

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 1 | Manual Handling Staff - could suffer from back pain if objects carried/moved incorrectly, there is repetition of the process and/or speed of the machine dictates production | 3 | 4 | 12 | - The machine is used for cutting profile metal to the desired length & width. The process is carried out on single profiles that can vary in weight and so no definitive number of staff required for lifting has been identified – staff MUST NOT lift any item beyond their own capability. Co-workers are located in close proximity and available to assist with lifting of metal profiles. - The metal profile is placed onto the machine bed by staff, ensuring that their own capabilities are not compromised. - Manual handling guidance provided within the induction process and informally on an ongoing basis by the Company Management. - Monitoring of work activities completed by the Factory Manager (IK) who ensures that manufacturing and fabrication processes include good manual handling practice. | 3 | 2 | 6 |  |
| 2 | Machine failure leading to potential impact on Staff e.g. electrocution Staff and others - could suffer electrical shock, burns or personal injury if equipment is faulty | 4 | 3 | 12 | - Pre-use maintenance check completed by the operator to ensure that the machine visually appears in good order and ready to use. - Concerns are raised with the Factory Manager (IK) who contacts external specialists for solutions. - Machines are subject to routine maintenance by appointed external specialists who visit on a regular basis. - Where required, machines that are awaiting repair or are damaged, are taken out of use and isolated at power sources to prevent use. - Machines are never used until issues or faults have been resolved appropriately. | 4 | 2 | 8 |  |

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 3 | Entrapment/Entanglement leading to cuts, abrasions, crush injury Staff could suffer personnel injury if they are were to become entangled in the machine whilst in operation | 4 | 4 | 16 | - Metal work staff completes daily checks of all machinery guarding to ensure that it is in place, working correctly and the machine is ready for use. - The cutting zone is protected by suitable light beam guarding, set 10mm below the upper plate, that allows the metal profile to be inserted to the cutting zone but automatically stops the machine cycle if the light beam is broken by anything. This ensures that the hands of the person using it are kept away from the cutting area. - The machine operation requires the operator to set the cutting requirements on the control panel, located to the side of the machine. Once set, the cutting cycle is automated and is started using a foot pedal(s). - Emergency stop button located on control panel and readily accessible. - The rear access to the machine is protected by light beam(s) guarding that prevents any person being able to access to, or be exposed to, moving parts - The manufacturer has identified that eye, hearing protection is required along with anti-cut gloves when handling profile metal. - Specific hearing protection has been established and provided for use with these machines. This is provided free of charge. - All staff that use this machine have been trained in-house by experienced operators. Records of training are held by the Company. - Manufacturer machinery guide is located in the office and can be referred to at any time if any concerns or issues are raised. | 4 | 2 | 8 |  |
| 4 | Unauthorised Use Staff could suffer personnel injury if they are using a machine that they are unfamiliar with | 3 | 3 | 9 | - Monitoring of work activities completed by the Factory Manager (IK) who ensure that good practice is followed. - The Factory Manager (IK) monitors the use of the machine on an ongoing basis to ensure that it is used correctly and to raise concerns or issues if others appear to be using it incorrectly. - Specific safety, training, machine authorisation and guidance documentation is readily available in the office. - Unauthorised use of any machine is deemed to be a direct violation of employee responsibility (section 7 HASAWA1974). Disciplinary action is used by the Company for any unauthorised use of machines. | 3 | 2 | 6 |  |

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 5 | Contact with substances/materials Staff could suffer occupational dermatitis, Asthma and other ill health effects caused by exposure to substances | 3 | 4 | 12 | - Material being cut (steel) and the method being used (guillotine) means that swarf or any airborne contaminants created by the cutting process are not created in any amount and not a significant issue. - Products/substances used during the machining process are limited to those required by the machine itself e.g. lubricants which are located within a reservoir that feed the machine operating parts and mechanics which is away from the operator. - Safety Data Profiles (SDS) for any hazardous substances used are held in the office and are available for review at any time. | 3 | 2 | 6 |  |
| 6 | Housekeeping Staff and others could suffer injury as a result of poor housekeeping creating hazards in the working area | 3 | 4 | 12 | - All staff are aware of their responsibility regarding the need for a high standard of housekeeping in their working area. - Waste generated is swept from the floor during the day and placed within specific bins in readiness for disposal and subsequent recycling if possible. - Off cuts are stored within the localised area and are used as and when possible (size dependent). - General waste disposed of in bins located throughout the premises for disposal. | 3 | 2 | 6 |  |
| 7 | Exposure to work-based noise Staff and others could suffer from noise induced hearing loss over a period of time if exposed to high levels of work-based noise | 3 | 4 | 12 | - Noise level readings vary dependent on size, structure of the profile being cut. - Review of noise levels completed previously by the Management Team using simple sound level meter (SSLM) to ascertain noise levels in areas of concern. - Machine operates at general levels of between 1st and 2nd action level (80 – 85dba) but with additional noise provided by other machines in close proximity. Machine operations are automatic, for short duration and not constant. - The manufacturer advises that the wearing of hearing protection is mandatory whilst machine is in use. - Hearing protection, in-ear buds & over-ear defenders are readily available via the Factory Manager (IK). All PPE, including hearing protection is provided to all fabrication staff free of charge. | 3 | 3 | 9 |  |

| ACTION ARISING FROM RISK ASSESSMENT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Risk Rating | Action Required: | Person (s) Responsible | Target Date | Date Completed |
| 1 | Low | In-house training/briefing on manual handling, fire safety awareness and this risk assessment to be provided to all fabrication staff. Monthly toolbox talks on relevant subject(s) to be issued to the fabrication staff | Jack Arnold Management | Annually Monthly |  |
| 2 | Low | Records of all machine maintenance, inspections and repairs to be held as an ongoing reference of the works completed on each machine. Information should be kept under review to ascertain if any trends occur such as breakdowns, frequent repairs of similar nature, replacement components etc. | Jack Arnold Management | Ongoing | N/A |
| 3 | Low | Company management to provide ongoing monitoring of work activities that all staff are following agreed safe methods of work and processes. | Jack Arnold Management | Ongoing | N/A |
| 5 | Low | Manufacturers/suppliers of products used by the Company to provide Safety Data Profiles (SDS). Latest SDS to be reviewed and CoSHH assessments to be reviewed annually (minimum) and updated as necessary. | Jack Arnold Management Assisted by MBHS Ltd |  |  |
|  | Various | Ensure that all signage relevant to the machine (mandatory PPE, Hazard warning, Prohibitive signs) is clearly visible and displayed in a prominent position to act as an ongoing reminder to the operator | Jack Arnold Management | Ongoing | N/A |
|  | Various | Ensure that the machine is subject to routine cleaning and operator maintenance checks so that it remains in good order. | Jack Arnold Management | Ongoing | N/A |
|  | Various | Ensure that the machinery undergoes routine maintenance, service and formal inspections by suitably competent persons (likely to be a 3rd party machinery specialist). The service intervals/requirements are to be as directed by the suitably competent persons. All inspections/maintenance to be documented and records held. | Jack Arnold Management | Ongoing | N/A |

Risk Assessment Acknowledgement

The following members of staff have read this risk assessment. 
Their signatures are confirmation that they have read and understood all which is within its contents.

| Name | Signature | Date | Name | Signature | Date |
| --- | --- | --- | --- | --- | --- |