| Area/Activity Assessed | Operating Welding equipment |  |  | Date | 30th October 2025 |
| --- | --- | --- | --- | --- | --- |
| Assessed by | Tony Smith-Allen | Assessed with | Lee Marden (Operations Manager) & Ilyan Karashtranov (Factory Manager) |  |  |

| Persons Exposed | Employees |  | Contractor |  | Young Person |  | Expectant Mother |  | Visitors and/or Public |  | Trespassers |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency of Exposure | Continually |  | Hourly |  | Daily |  | Weekly |  | Monthly |  | Yearly |  |
| Duration of Exposure | Less than 1hr |  | 1-2 hrs |  | 3-4 hrs |  | 5-6 hrs |  | 7-8 hrs |  | More than 8 hrs |  |

| Probability (Prob) | 5=Very Likely, 4= Likely, 3= Quite Possible, 2= Possible, 1= Unlikely |  | 0-8 - Low risk No Action Required. 9-15 - Medium risk Ensure adequate controls are in use. 16-25 - High Risk Stop operation and implement adequate control measures |
| --- | --- | --- | --- |
| Severity (Sev) | 5=Catastrophic, 4=Major, 3=Moderate, 2=Minor, 1=Insignificant |  |  |

This Risk Assessment & Control Measures has been established following consultation with Management, Supervisory and Operational staff.

Staff acknowledgment as to their understanding of the content and the control measures to be applied is included as the last page to this document

| Hazard |  | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
|  | Unauthorised use and operation of equipment Operator could suffer severe personal injury or cause injury to others as a result of using a piece of equipment that they are unfamiliar with | 4 | 4 | 16 | - Monitoring of work activities completed by the Factory Manager (IK) who are located in the working areas throughout the day and ensure that good practice is followed. - Only trained competent staff utilise equipment and a Competency Matrix has been established by the Company to ensure clarity of who is authorised to use named pieces of equipment. - Specific safety, training, equipment authorisation and guidance documentation is readily available from the Company. - The welding package & shielding gas cylinders are stored in a secure location within the boundaries of the company premises. The welding sets are secured within the factory, and the shielding gas secured in close proximity to the welding bays. - Empty gas cylinders are removed/exchanged by the supplier as and when requested. - Users are not permitted to operate the equipment alone; the company operate a ‘buddy system’ so that there is always at least 2 people. - Unauthorised use of any tool or piece of equipment is deemed to be a direct violation of employee responsibility (section 7 HASAWA1974). Disciplinary action is used by the Company for any unauthorised use of tools and equipment. | 4 | 2 | 8 |  |

| Hazard |  | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
|  | Incorrect storage Staff and 3rd parties could suffer severe personal injury or worse as a result of poor storage arrangements, empty cylinder abuse | 3 | 3 | 9 | - Shielding gas cylinder valves are closed when not in use. - Empty cylinders are treated as though full, applying the same precautions e.g keeping them upright, secured etc. - Cylinder key is removed by a member of the welding team. - Empty cylinders are stored in designated area outside of the premises and are returned to gas supplier or agent as soon as possible. - The position chosen for storage is levelled using stabilising timbers to offset the floor camber and provides a suitable load-bearing surface. - Cylinder(s) secured in a suitable cylinder pen that is outside. - Cylinders are stored with their valves uppermost. This ensures that if there is a leak from the valve area, vapour and not liquid will escape. - The power unit is turned off and unplugged when not in use. - The torch, hoses and pressure regulator are vented, as per equipment instructions. - Gas hoses and electric cables are rolled up are stored on the appropriate stowage point on the package. - The welding package & shielding gas cylinders are stored in a secure location away from flammable/combustible materials and sources of ignition. - Warning notices in place advising of works being done. - Where possible cylinders are always stored in the dedicated outdoor area, and cylinders brought indoors as needed – this is for the current set-up of additional welding done with the window manufacturing area. | 3 | 2 | 6 |  |
|  | Manual handling Operator could suffer personal Injury due to incorrect handling. Poor handling could also result in property damage | 3 | 3 | 9 | - Large cylinders are moved using a suitable trolley or assistance is sought. - Large gas cylinders are uncoupled from welder and moved separately to avoid an unstable load. - Operators are advised to not drop gas cylinders and never try to catch a falling cylinder. - Cylinders are secured in a suitable cylinder trolley, cylinder support bracket or placed in a cylinder pen. | 3 | 2 | 6 |  |

| Hazard |  | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
|  | Incorrect use of MIG welding package and equipment Staff could suffer severe personal injury or worse due to electrocution or Fire resulting from the misuse of the MIG welder | 4 | 4 | 16 | - Welding equipment conforms to the appropriate international (ISO) or British (BS) standards. - All users are trained/instructed/competent in: - Properties of the shielding gas. - The safe use of the equipment. - Precautions to be taken. - The use of fire extinguishers. - Emergency Procedures. - Pre-use equipment checks are carried out on the welding package equipment being used. Torch nozzle and tip are cleaned before use. - Procedures/work instructions for the safe use of the equipment are provided. - Designated area for welding operations (i.e. factory) or used under ‘Hot work’ permit. - The following work rules have been created and are followed: - FORBIDDING the use of the equipment where wet or damp conditions exist (e.g. in the rain). - FORBIDDING the positioning of the welder on the metal work piece. - A safe earth connection, using a clamp to the work piece, is established. - The current return cable is clamped to a clean metal surface close to where the operator is welding. Stray currents are more likely if the welding return path exhibits a high electrical resistance e.g. the return is clamped onto a rusty surface. - Electric cables and gas hoses are positioned out of the line of fire from the torch or metal spatter. - The correct pressures and nozzle size for the job are used. - Correct settings are established using the manufacturer’s guidance, i.e. gas pressure, electrode speed, power. - The power unit’s air vents are kept clear of all obstructions when in use. - The user of the equipment must remove all personal jewellery, especially rings, bracelets and metallic watch straps, before carrying out any welding (welding currents can induce currents in these objects high enough to cause partial melting). - Appropriate PPE is provided and worn, i.e. insulated leather welding gloves, leather apron, suitable cotton coverall, foot protection, eye and head protection. - Respiratory protection is provided by the welder wearing an “air-fed” mask. | 4 | 2 | 8 |  |

| Hazard |  | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
|  | Use of faulty or incorrect equipment (gas regulator, torch, welding package, hoses, electric cables etc.) resulting in fire or electrocution Operator and others could suffer severe personal injury or worse resulting from the use of faulty or incorrect equipment (gas regulator, torch, welder, hoses, electric cables etc.) | 4 | 4 | 16 | - Pre-use equipment checks are carried out on the MIG welding package and equipment, including: - Welder. - Torch nozzle and tip are cleaned and greased before use. - Welding wire (electrode). - Hose assembly. - Mains cable. - Connectors (ensuring they are correctly rated for the current required). - Earth clamp. - Gas cylinder regulator. - Procedures/work instructions for the safe use of the equipment are provided. - Gas cylinder regulator is refurbished or replaced every 5 years. - Thermal overload protection device with auto reset function. - Defective equipment is replaced before attempting to use the MIG welding package. | 4 | 2 | 8 |  |
|  | Accidental ignition due to poor housekeeping, work area design, or inspection Operator and 3rd parties could suffer severe personal injury or worse as a result of not following safe working practises | 5 | 3 | 15 | - The immediate work area is cleared of any flammable materials and or suitable fire screens or blankets put in place to prevent contact between sparks and flammables. - A work site inspection is carried out after completion of hot work to ensure there are no residual hot spots or smouldering fires that could escalate to a large fire. - Appropriate fire extinguishing appliances are provided. | 5 | 2 | 10 |  |
|  | Welding arc-eye The UV and infrared light generated by MIG welding is extremely bright. Looking directly at a welding arc even for a short time can cause ‘arc-eye’ when the bright flash from the arc burns the cornea | 4 | 5 | 20 | - An appropriate full face welding helmet is worn, and the lens intensity adjusted to suit. - Welding takes place in a designated area where welding screens/curtains are erected/drawn to protect others from harmful exposure. - Personnel in the vicinity of a welding operation are warned of the hazard, the operator is not permitted to commence welding without ensuring the safety of other personnel first. | 4 | 2 | 8 |  |

| Hazard |  | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
|  | Metal fumes Exposure to harmful welding fumes can result in ill health (pneumonia, metal fume fever, cancer, occupational asthma) | 4 | 4 | 16 | - Welding is carried out in a well-ventilated area/operator will ensure adequate ventilation of the working area. - Work process restricts airborne contaminants to localised area (e.g. designated area) and not a significant issue for non-operational personnel. - Weld fume is minimal with MIG welding, but “on torch” extraction and suitable RPE (Air Fed Welding Helmet) is used as the primary means of removal of airborne contaminants. - Operator will ensure that materials to be welded are clean, i.e. free from harmful coatings, this provides mitigation against the creation of harmful airborne contaminants. | 4 | 2 | 8 |  |
|  | Protection from UV and infrared light, molten metal (splatter), and hot work piece The light from MIG welding has a strong ultraviolet and infrared content and causes ‘sunburn’. Blobs of molten metal can burn through any thin clothing and cause small but painful burns | 4 | 4 | 16 | - Appropriate PPE is provided and worn, i.e. insulated leather welding gloves, leather apron, suitable cotton coverall, foot protection, eye and head protection. - Full covering of arms and legs is essential, in hot weather operator wears thin cotton clothes in preference to stripping to exposed skin. - The operator ensures that there are no gaps in their protective clothing or shoes where a blob of metal could enter. - The metal being welded stays hot for a good while. Operators shall exercise extreme care when handling a recently welded work piece with their bare hands. - After use, welded materials are put in a safe place to cool down or the welding area is protected, so as to minimise the risk of fire or burns. | 4 | 2 | 8 |  |

| ACTION ARISING FROM RISK ASSESSMENT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Risk Rating | Action Required: | Person (s) Responsible | Target Date | Date Completed |
| All | Low | In-house training/briefing on Respiratory Protection, manual handling, fire safety awareness and this risk assessment to be provided to all fabrication staff. Monthly toolbox talks on relevant subject(s) to be issued to the fabrication staff | Jack Arnold Management | Annually Monthly |  |
| All | Low | Management to provide ongoing monitoring of work activities that all staff are following agreed safe methods of work and processes. | Jack Arnold Management | Ongoing | N/A |
| 4/5 | Low | Records of all machine maintenance, inspections and repairs to be held as an ongoing reference of the works completed on each machine. Information should be kept under review to ascertain if any trends occur such as breakdowns, frequent repairs of similar nature, replacement components etc. | Jack Arnold Management | Ongoing | N/A |
| 6 | Medium | Fire extinguishers to be readily available and in close proximity of every welding bay. Consideration to be given to the use of appropriate P50 style extinguishers | Jack Arnold Management | Within 6 months | N/A |
| 8 | Low | Manufacturers/suppliers of products used by the Company to provide Safety Data Sheets (SDS). Latest SDS to be reviewed and CoSHH assessments to be reviewed annually (minimum) and updated as necessary. | Jack Arnold Management Assisted by MBHS Ltd | Within 6 months |  |
|  | Various | Ensure that all signage relevant to the machine (mandatory PPE, Hazard warning, Prohibitive signs) is clearly visible and displayed in a prominent position to act as an ongoing reminder to the operator | Jack Arnold Management | Ongoing | N/A |
|  | Various | Ensure that Respiratory Protection and other PPE is worn by the operator as directed. | Jack Arnold Management | Ongoing | N/A |
|  | Various | Management to implement and maintain weekly routine & inspections of the premises (and works being completed) so that a record of any actions or required improvements can be held. | Jack Arnold Management | Weekly | N/A |

Risk Assessment Acknowledgement

The following members of staff have read this risk assessment. 
Their signatures are confirmation that they have read and understood all which is within its contents.

| Name | Signature | Date | Name | Signature | Date |
| --- | --- | --- | --- | --- | --- |