| Area/Activity Assessed | Operating Forklift Truck | Date | 30th October 2025 |
| --- | --- | --- | --- |
| Assessed by | Tony Smith-Allen | Assessed with | Lee Marden (Operations Manager) & Ilyan Karashtranov (Factory Manager) |

| Persons Exposed | Employees |  | Contractor |  | Young Person |  | Expectant Mother |  | Visitors and/or Public |  | Trespassers |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency of Exposure | Continually |  | Hourly |  | Daily |  | Weekly |  | Monthly |  | Yearly |  |
| Duration of Exposure | Less than 1hr |  | 1-2 hrs |  | 3-4 hrs |  | 5-6 hrs |  | 7-8 hrs |  | More than 8 hrs |  |

| Probability (Prob) | 5=Very Likely, 4= Likely, 3= Quite Possible, 2= Possible, 1= Unlikely |  | 0-8 - Low risk No Action Required. 9-15 - Medium risk Ensure adequate controls are in use. 16-25 - High Risk Stop operation and implement adequate control measures |
| --- | --- | --- | --- |
| Severity (Sev) | 5=Catastrophic, 4=Major, 3=Moderate, 2=Minor, 1=Insignificant |  |  |

This Risk Assessment & Control Measures has been established following consultation with Management, Supervisory and Operational staff

Staff acknowledgment as to their understanding of the content and the control measures to be applied is included as the last page to this document

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 1. | People movement throughout internal and external areas. Staff and/or 3rd parties could suffer personal injury or worse if they were to collide with FLT moving throughout internal/external areas | 4 | 4 | 16 | - FLT is moving/arranging loads etc. away from the general staff within the factory. - Finished goods are located away from staff and as close to the full height shutter(s). - Adequate lighting levels in place where there is regular movement of vehicles, potential pedestrians and building access points. - FLT are fitted with audible warning devices e.g. automatic reversing bleepers, flashing warning beacons. These are used at agreed locations e.g. turning into blind corner, reversing etc. - Visibility aids are used where required e.g. convex mirrors at blind corners. - High-visibility clothing is worn by pedestrians where the risks to their safety cannot be adequately controlled by other methods. - Clear, easy read warning signs are displayed in prominent positions to inform people that FLT’s are in operation in and around the premises and external areas. - FLT drivers utilise a banksman if required to manage pedestrians when loading vehicles (ensuring that lorry drivers are located away from FLT movements during the loading/unloading process) | 4 | 3 | 12 |  |

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 2. | FLT tipping over due to unbalanced load Driver could suffer personal injury or worse if the FLT were to tip over | 4 | 3 | 12 | - FLT drivers are suitably trained/competent in ensuring that the load they are carrying is balanced. This training includes: - Competent use of controls, - Movement of the truck with forks or attachments in the correct travel position, laden and unladen, - Correct insertion and withdrawal of forks or other handling attachments without damage to the pallet or load, - Manoeuvring a laden and unladen truck forward and in reverse in a narrowly confined area, - Performing both a left and a right 90º turn with a loaded truck. - Fork tines are lowered when the truck is on route to required destination therefore reducing centre of gravity and tipping point of the load. - FLT is fitted with roll cage protection. - Driver wears seat belt whilst operating FLT. - Loads are palletised by the supplier to SBF so as to ensure secure loading onto the tines. - Defective pallets are not unloaded where their stability can’t be assured. The load may need to be re-packaged, before the FLT can move it. - Defective pallets are removed from use where possible. | 4 | 2 | 8 |  |
| 3. | FLT tipping over due to excess speed Driver could suffer personal injury or worse if the FLT were to tip over | 4 | 4 | 16 | - Drivers are suitably trained/competent to operate the FLT they have been instructed to use. This training includes: - Competent use of controls, - Movement of the truck with forks or attachments in the correct travel position, laden and unladen. - FLT drivers are trained/experienced working at the premises. All FLT drivers (5 people) are aware that slow speed is required when using the FLT. - Management monitor speeds and general work observations using the CCTV that covers the internal/external areas and give verbal warnings if necessary. Repeat offenders are removed from using FLT and subject to company disciplinary procedures. - FLT is fitted with roll cage protection. - Driver wears seat belt whilst operating FLT. | 4 | 3 | 12 |  |

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 4. | FLT tipping over due to ground conditions of operating areas Driver could suffer personal injury or worse if the FLT were to tip over | 4 | 3 | 12 | - Flooring/ground conditions subject to ongoing review (visual inspection) by management and FLT drivers. Any changes in conditions e.g. cracking, pot holes etc. are reported immediately to the Company management team who arrange for suitable repair to be completed. - Accumulations of loose material e.g. stones, gravel etc. on the ground are cleared on a regular basis as they are particularly hazardous to small-wheeled lift trucks. - The Company have removed gradients along FLT routes where possible. Drivers never allow FLT to be driven up or down gradients that exceed the maximum specified by the manufacturer or authorised supplier. - Drivers are suitably trained/competent to operate the FLT they have been instructed to use including never travelling or turning a FLT across a gradient. | 4 | 2 | 8 |  |
| 5. | Collision with other vehicles e.g. delivery/collection vehicles, company vehicles, staff cars Staff and/or other drivers could suffer personal injury or worse if their vehicle collided with a FLT moving through the external roadways | 4 | 3 | 12 | - Arriving delivery drivers or those company drivers awaiting loading in readiness for their daily route remain within the cab of their vehicle when the FLT is in operation. If not, they are required to be inside the building or away from all routes the FLT may take. - Informative notices instructing FLT drivers to sound horns at appropriate locations e.g. blind corners, entering buildings. - FLT’s are fitted with audible warning devices e.g. automatic reversing bleepers, flashing warning beacons. - High-visibility clothing is worn by pedestrians where the risks to their safety cannot be adequately controlled by other methods. - FLT drivers wear high-visibility clothing at all times so they can be clearly seen when leaving the operating position of the truck. | 4 | 2 | 8 |  |
| 6. | Collision with buildings/access openings Driver could suffer personal injury or worse if the FLT collided with the building(s) or access openings | 4 | 3 | 12 | - The Company make sure that roads and aisles have sufficient width and overhead clearance for the largest FLT to operate safely, whether loaded/unloaded, and if necessary, to allow other vehicles/loads to pass each other safely. - One way system is used where possible, e.g. defined and differing entry and exit points. | 4 | 2 | 8 |  |

| No | Hazard | Initial |  |  | Existing Control Measures | Residual |  |  | Extra Controls |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Sev | Prob | Risk |  | Sev | Prob | Risk |  |
| 7. | FLT mechanical failure Driver could suffer personal injury if the mechanical operations of the FLT were to fail whilst in use | 3 | 3 | 9 | - Daily checks are completed by FLT drivers before their shift starts. Any faults are reported to the Company supervisor who will arrange for suitable repair. - Maintenance regime/contract in place and managed by the Company management team. This ensures that the FLT’s are subject to servicing and preventative maintenance on a regular basis. - Thorough Examination completed by a competent person in line with statutory requirements (LOLER98) - FLT’s maintained by supplier (under contract) on a regular basis. | 3 | 2 | 6 |  |
| 8. | Parking and storage of fork truck Staff and/or 3rd parties may suffer injury due to tripping over the fork tines while it is parked up | 3 | 3 | 9 | - The Company have established suitable storage/parking areas for FLT’s. These locations are away from areas where potential conflict or unauthorised access could occur. - FLT drivers are trained on safe parking procedures (e.g. park truck away from pedestrian access and gangways, fork tines placed flat on the floor with a slight downward angle so ends of fork tines are flush with the floor) | 3 | 2 | 6 |  |
| 9. | Manual Handling Driver could suffer back injury when moving the tines along mast track, removing forks from mast track etc | 3 | 3 | 9 | - Removal of fork tines is only carried out by the Company appointed service/maintenance contractor. - Manual handling training/briefing is provided for FLT drivers to remind them of best practice when moving the fork tines. - Tracks lubricated as part of the ongoing routine maintenance to assist with fork tine movement. - Location lugs and catches are maintained and serviced which makes the operation easier for the FLT driver. - Safety boots and gloves issued to all FLT drivers for use when moving the fork tines. | 3 | 2 | 6 |  |
| 10. | Changing LPG cylinder Driver(s) and/or 3rd parties could suffer personal injury or worse if an incident were to occur whilst changing the LPG cylinder on the FLT | 4 | 3 | 12 | - LPG cylinders are changed outside of the factory and away from the opening shutter. - 3rd parties do not have access as the area used is located away from general areas required by delivery/collection vehicles. - Area kept clean, tidy and maintained as necessary by in-house staff. - Cylinders are switched off at the valve before disconnected from the FLT. The FLT manual provides clear instruction on changing the cylinder, which is followed by the driver. - No smoking around the area where cylinders are being changed. | 4 | 2 | 8 |  |

| ACTION ARISING FROM RISK ASSESSMENT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Risk Rating | Action Required: | Person (s) Responsible | Target Date | Date Completed |
| All | Various | FLT drivers to receive refresher training suitable for their knowledge and experience. Frequency of training to be in line with RTITB guidance. | Jack Arnold Management | Ongoing |  |
| 1 | Low | Staff to be reminded that they must keep away from areas where the FLT are operating. FLT drivers to be reminded that they must stop immediately when a person approaches them. | Jack Arnold Management | Ongoing |  |
| 4 | Low | Ongoing visual inspections to be completed of all FLT routes and floor surfaces. Any significant damaged areas e.g. large cracks, pot-holes etc, to be subject to planned maintenance/repair with a longer-term view of upgrading/resurfacing where required. | Jack Arnold Management | Ongoing |  |
| 7 | Low | FLT drivers MUST complete a daily pre-use checklist that provides an overview of the vehicle condition. Any defect or area of concern MUST be reported to the management team. | Jack Arnold Management | Ongoing |  |

Risk Assessment Acknowledgement

The following members of staff have read this risk assessment. 
Their signatures are confirmation that they have read and understood all which is within its contents.

| Name | Signature | Date | Name | Signature | Date |
| --- | --- | --- | --- | --- | --- |